3GPP TSG-SA3 Meeting #117	S3-242907

Maastricht, the Netherlands, 19th - 23th August 2024

# Source:	ZTE Corporation

# Title:	MAC CE integrity protection solution for LTM

# Document for:	Approval

Agenda Item:	4.12

1	Decision/action requested

This contribution proposes a new solution for LTM.

2	References

[1]	S3-241773 (R2-2404037) LS on security handling for inter-CU LTM in non-DC cases

3	Rationale

This contribution proposes a new solution for LTM by enhancing the option 1A in [1].

As discussed in S3-242906, since  information is transmitted in plain text in the MAC CE, NCC value or the index of the NCC value included in the  LTM cell switch command MAC CE is possible to be illegally tampered. This contribution proposes a solution to provide integrity protection for MAC CE.

4	Detailed proposal

**** Start of Changes****

3.Y	Solution #Y: MAC CE integrity protection for LTM

3.Y.1	Introduction

This solution is for key issue #X. This solution is an enhancement of option 1A where the NCC value is included in the cell switch command MAC CE. The proposal is to provide integrity protection for the cell switch command MAC CE. The integrity protection token is similar to shortMAC-I or ResumeMAC-I currently used in the RRCReestablishmentRequest message and RRCResumeRequest message, respectively.

- 3.Y.2	Solution details

To protect the integrity of the MAC CE, an integrity protection token is included in the cell switch command MAC CE.  In order to calculate the token, the source gNB uses the integrity algorithm in the 5G AS security context with the following parameters as input:

-the last serving cell PCI (i.e. source cell PCI);

-the last serving cell C-RNTI (i.e. source cell C-RNTI);

-the cell ID of of the target cell (e.g. CGI, PCI, cell identity, the candidate configuration index);

-KEY shall be set to the integrity protection key of the target cell, e.g. the integrity protection key for the control plane KRRCint derived based on the NCC included in the MAC CE ;

-all BEARER bits shall be set to 1;

-DIRECTION bit shall be set to 1;

-all COUNT bits shall be set to 1;

MESSAGE bit shall be set to the bit length/size of the MAC CE, or the MAC CE plus its subheader.

Upon receiving the cell switch command, the UE derives new KgNB based on the received NCC value, i.e, the UE first synchronize the locally kept NH parameter iteratively and increasing the NCC value until it matches the NCC (if the received one is different from the NCC value associated with the currently active KgNB) and computes the KNG-RAN* from the synchronized NH parameter. Then the UE checks the integrity of the received MAC CE based on the new derived key and the integrity protection token. If the integrity check fails, the UE discards the MAC CE and initiates the RRC re-establishment procedure. If the integrity check success, the UE performs the LTM cell switch execution as existing procedure.

3.Y.3	Evaluation

TBD

**** End of Changes****