3GPP TSG-SA3 Meeting #117	S3-242957

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Intel

# Title:	Cleanup TR 33.754 Introduction

# Document for:	Approval

Agenda Item:	5.15

# 1	Decision/action requested

It is requested to approve the cleanups to TR 33.754.

# 2	References

# 3	Rationale

Cleanup of TR 33.754 as following SA2's interim conclusion communicated via S2-2407145. SA2 concluded that there is no consensus to proceed with normative work for Key Issue #2.2 in Release 19, which directly impacts the FS_MASS_SEC study by leaving it without specific security requirements to address.

# 4	Detailed proposal

SA3 is requested to approve the following pCR.

*************** Start of Changes **********************

# Introduction

# 1	Scope

The present document aims to address the security aspects of Multi-Access, focusing on DualSteer devices and ATSSS Phase 4 (ATSSS Ph-4) enhancements, as identified in the FS_MASSS study in TR 23.700-54 [2].

The scope of this study includes the following key areas:

-	Examine the security implications of a simplified architecture over non-3GPP access, where non-3GPP access does not rely on the current TNGF/N3IWF architecture. This involves assessing:

-	Whether to keep NAS security context on non-3GPP access.

-	Whether to keep IPsec on the user plane and/or control plane of non-3GPP access.

-	Whether new security mechanisms are to be considered in UE procedures, particularly regarding registration and connectivity to the 5G system in the context of ATSSS between 3GPP and non-3GPP access without 5G NAS.

The study will leverage insights from the requirements and architectural considerations outlined in the FS_MASSS study in TR 23.700-54 [2].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-54 "Study on Multi-Access (DualSteer and ATSSS_Ph4)".

[3]		     3GPP TS 33.501: " Security architecture and procedures for 5G system"

[4]	3GPP TS 23.501: "System architecture for the 5G System (5GS) "

[5]	IETF draft-ietf-quic-multipath-07: "Multipath Extension for QUIC".

[6]	RFC 9000; "QUIC: A UDP-Based Multiplexed and Secure Transport"

[7]	RFC 9001: "Using TLS to Secure QUIC"

[8]	draft-ietf-quic-multipath-07 "Multipath Extension for QUIC "

[9]	RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3"

[10]	RFC 9001: "Using TLS to Secure QUIC"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Non-Integrated Non-3GPP Access: as defined in TR 23.700-54[2].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

Void

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

NIN3A		Non-Integrated Non-3GPP Access
ATSSS		Access Traffic Steering, Switching, and Splitting
MP-QUIC	Multipath QUIC

# 4	Security assumptions

Void

*************** End of Changes **********************