3GPP TSG-SA3 Meeting #117	S3-242962

Maastricht, Netherlands  19 - 23 August 2024

Source:	NEC

Title:	Discussion on Dual steer device and AKMA procedure

Document for:	Approval

Agenda Item:	5.15

Work Item / Release:	Study on security aspects for Multi-Access/ Rel-19

Abstract of the contribution: This contribution porposes a study to investigates the impact of the dual steer feature on the AKMA procedure.

1. Introduction

This proposes an objective to the to study the impact of dual steer feature on AKMA procedure.

2. Discussion

As identified in the SA1 study and service requirements on DualSteer (see TR 22.841 and TS 22.261), it is beneficial for a DualSteer Device to apply traffic steering and/or switching between two 3GPP access networks connected to the same or different PLMN networks. Use cases cover examples of diverse combinations of 3GPP access networks using the same or different RATs, including terrestrial NR plus NR, or NR plus E-UTRA (e.g. using a combined EPC and 5GC), a mix of terrestrial and non-terrestrial NR, as well as dual non-terrestrial NR access (using same or different NTN orbits, e.g., GEO/MEO/LEO).

The introduction of DualSteer feature can have impact on the AKMA procedue. The following discussion tries to capture the impact of DualSteer feature on the AKMA functionality.

1. Architecture of the dual steer device.

Below is simple architecture of the dual steer UE. It consists of two Mobile Terminals one with SUPI 1 and another with SUPI2, operating system and application. The two SUPIs should be correlated in order to get the dual steer service.

Operating System

App1

App2

APP3

UE 1(SUPI 1)

UE 2(SUPI 2)

Dual Steer UE

Dual steer layer

Figure 1: Architecture of dual steer UE.

2. Non roaming architecture of the network supporting dual steer operation.

Below is the simple example of the network supporting dual steer operation in non-roaming case. In order to support dual steer operation, the SMF and UPF will be common to both PDU Session of UE1 and PDU session of UE 2 for a S-NSSAI and a DNN combination.

Figure 2: Non-Roaming architecture of the network supporting dual steer operation.

3. Signalling flow for the Dual steer operation.

The signalling flow below gives a very simple example how the dual steer will work.

Figure 3: Examples on DualSteer Procedures

1. The UE 1 will be registered to the PLMN via AMF1 Kausf1, Kakma1 and Kaf1 are derived after the registration procedure of the UE 1.

2. The UE1 establishes a PDU session 1 for the S-NSSAI and the DNN. Dual steer rules can be given at this time.

3.The UE 2 is registered to the PLMN via AMF2. Kausf2, Kakma2 and Kaf2 are derived after the registration of USIM 2.

4. The UE 2 establishes a PDU session 2 for the S-NSSAI and the DNN. The dual steer rules can be given at this time or the dual steer rules given in step 2 can be updated.

5. An application (e.g., APP 1) starts using transmitting user traffic over the UE1 (PDU session 1) providing security using Kaf1 to the server handling App1.

6. Suppose UE1 lost coverage of the network. Then according to the dual steer rule, the DS stack will move the user traffic from UE1 (PDU session 1) to UE2 (the PDU session 2).

In step 6 following things are not clear:

1. When the user traffic of App 1 moves from UE1 (PDU Session 1) to UE2 (PDU session 2) by dual steer rules, the does the UE operating system or the App1 need to aware if the user traffic of App1 is switched from UE 1 (SIM 1) to UE 2 (SIM 2)?

2. If the application App1 knows that user traffic is changed from UE1 (PDU Session 1) to UE2 (PDU session 2) then the security protection of user plane of App1 can be changed using Kaf1 to Kaf2 or user plane traffic of App 1 will remain using the Kaf1 for the security protection?

3. If the user traffic of App1 is changed from UE1 (PDU Session 1) to UE2(PDU session 2), how does the AF know the user traffic of the App1 has been changed from UE 1 to UE 2 ? how does an AF knows which security keys will be used for the user traffic via UE2 (PDU Session 2)?

4. The same issue is expected for the GBA and GAA procedure.

3. Proposal

It is proposed to study the impact of dual steer feature on AKMA procedure. An objective and key issue to study the impact of  dual steer feature on AKAM procedure is proposed in SA3 SID DualSteer + ATSSS Ph-4. The related contribution is submitted in S3-242960.

* * * Start of Change * * *