3GPP TSG-SA3 Meeting #117	S3-242988

Maastricht, NL, 19 – 23 Aug 2024

# Source:	Nokia

# Title:	New key issue for satellite switching with re-synchronization in regenerative architecture

# Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

Approve the new key issue proposed.

# 2	References

[1]	3GPP TR 33.700-29 Study on Security Aspects of 5G Satellite Access in the 5G Architecture phase 3

[2]          3GPP TR 23.700-29 Study on integration of satellite components in the 5G architecture

# 3	Rationale

As part of Rel-18, RAN2 discussed the specification of the satellite switching with resynchronization feature. This feature, also known as “unchanged PCI” mobility, allows a UE to switch from one satellite to another one without carrying out an L3 handover (i.e., no dedicated RRC signalling is involved) under the conditions that gNB (which does not change) is on the ground and satellites will work as bent-pipe network nodes (i.e., transparent payload). Since the gNB does not change, the cell configuration can be maintained (i.e. SSB, SIB, paging and UE’s RRC configuration) after the satellite switching and no path switch is conducted at the AMF/UPF.

In a regenerative architecture, the satellite movement will constantly trigger inter-gNB mobility events (gNB on board of the satellite). This contribution proposes a new key issue for NTN “unchanged PCI” (Earth Fixed cell) in regenerative payload architecture. NTN may be deployed either to have: Earth Fixed Cells (EFCs) – wherein the satellite beam is steered on to a geographical NTN cell on earth from minimum elevation angle on one side of the horizon to the minimum elevation angle on the other side of the horizon. Or Earth Moving Cells (EMCs) – wherein the satellite beam shall be fixed and not steerable and moves as the satellite moves.

Considering NTN deployments with Earth Fixed Cells (EFCs) as it is less complex deployment, it may be realized via one of the following options:

Option 1: Satellite switch results into PCI change. The likely realization of this is to associate each NTN cell with a pair of PCIs, that change alternatively with satellite switch.

Option 2: PCI is unchanged due to satellite switch.

In Rel-18, RAN2 agreed to support ‘Satellite switching without PCI change’ for the case of Quasi Earth fixed cell involving hard satellite switch with same gNB for transparent mode of satellite operations.

Option 2 could be discussed for regenerative NGSO scenarios in Rel-19 for scenarios involving satellite switch involving gNB change.

Below are few contributions discussed in RAN2 for Earth Fixed cell.

- R2-2405021

- R2-2405126

- R2-2402844

- R2-2402808

The security aspects related to regenerative NGSO scenarios should be studied in SA3, and hence this proposal for a new KI.

# 4	Detailed proposals

# * * * * First change* * * *

## 5.x	Key Issue#x: Security impacts for satellite switching with re-synchronization in regenerative architecture

### 5.x.1	Key issue details

For the satellite switching with re-synchronization, the PCI of NTN cell is fixed for stationary UEs and for the UEs that do not undergo cell change, even though the serving satellite (and thus the serving gNB) changes after every fixed period of time. The security aspects of such satellite switching in a regenerative architecture (with unchanged PCI) needs to be studied. From security perspective, this would mean that the current set of parameters used in key computation at source gNB are : target cell PCI, target ARFCN-DL/EARFCN-DL and the KgNB/NH. Also, for authentication token at the UE (at the time of connection resume): source PCI, target Cell-ID, source C-RNTI, may introduce security risk because PCI is the same for the last serving gNB and the current serving gNB.

For the UEs in RRC_CONNECTED experiencing such satellite switch during resync for unchanged PCI, or for the UEs in RRC_INACTIVE state moving to RRC_CONNECTED state via a different satellite, the UE may not be aware of the security key derivation principle used at the radio access network, for e.g. mobility between NTN-NTN with EFC and unchanged PCI or the security key derivation method of the mobility for e.g. between NTN-TN where the target cell is TN.

### 5.x.2	Security threats

Source and target gNB might have same keys during satellite switch during resync in regenerative architecture for unchanged PCI and the traffic could be monitored by any Satellite.

### 5.x.3	Potential security requirements

The 3GPP system shall support means to provide confidentiality, integrity for earth fixed cell scenarios for user-plane and control-plane messages between UE and the 3GPP network in the regenerative satellite operations in case of satellite switching with resynchronization.

# * * * * End of changes * * * *