3GPP TSG-SA3 Meeting #117 	S3-242997

# Maastricht, NL, 19 - 23 August 2024

# Source:	Apple

# Title:	MAC CE based solution for LTM security

# Document for:	Approval

Agenda Item:	4.12

# 1	Decision/action requested

Approve the pCR to the living CR of LTM security

# References

# 3	Rationale

This contribution proposes a new LTM security solution, which is based on MAC CE to deliver NCC.

# 4	Detailed proposal

*** Start of 1st Change ***

## 3.X	Solution #X: MAC CE based solution to deliver the NCC(s)

### 3.X.1	Introduction

This solution proposes the procedure for LTM key derivation. The legacy handover procedure in TS 33.501 clause 6.9.2 will be reused as much as possible. RAN2 procedures on LTM (no DC) are used as the basis.

### 3.X.2	Details

Preassumption:

- In this solution, gNB1, gNB2 and gNB3 are the candidate cells, UE is moving from gNB1 to gNB2, then from gNB2 to gNB3.

- Source gNB1 configures the UE, gNB2 and gNB3 with the list of NCCs and the corresponding index/identifier to be used for the key derivation.

- Every NCC has an index/identifier, the Index/identifier is used to conceal the NCC.

- AMF needs to be configured with this list of NCC and Index/identifier.

Figure 3.x.1-1. MAC CE based LTM procedure

Step 0. Source gNB1 configures the UE, gNB2 and gNB3 with the list of NCCs and the corresponding index/identifier to be used for the key derivation. Every NCC has an index/identifier, the Index/identifier is used to conceal the NCC. AMF needs to be configured with this list of NCC and Index/identifier.

Step 1. gNB1 sends the MAC CE to the UE including the Index/identifier of the NCC1(assuming NCC1 is the next NCC to be used for next handover.

Step2. gNB1 derives K_gNB21 based on NCC1 and sends it to gNB2, the key derivation method follows TS 33.501 Annex A.11.

Step3. gNB1 derives K_gNB31 based on NCC1 and sends it to gNB3, the key derivation method follows TS 33.501 Annex A.11.

NOTE: the order of step 2/3 and step1 follows RAN2 procedure.

Step4. UE triggers the inter-CU switch, hanover from gNB1 to gNB2. UE derives K_gNB21 using NCC1.

Step5. After the handover, gNB2 sends NGAP PATH SWITCH COMPLETE message to AMF, including the NCC1 being used.

Step6. AMF sends the NGAP PATH SWITCH ACK message back to gNB2, including a new pair {NH2, NCC2}.

Step7. gNB2 derives K_gNB12 based on NCC2 and sends it to gNB1, the key derivation method follows TS 33.501 Annex A.11.

Step8. gNB2 derives K_gNB32 based on NCC2 and sends it to gNB3, the key derivation method follows TS 33.501 Annex A.11.

Step9. gNB2 sends the MAC CE to the UE including the Index/identifier of the NCC2 to be used for next handover.

NOTE: the order of step 7/8 and step9 follows RAN2 procedure.

Step10. UE triggers the inter-CU switch, hanover from gNB2 to gNB3. UE derives K_gNB32 using NCC2.

Step11. After the handover, gNB3 sends NGAP PATH SWITCH COMPLETE message to AMF, including the NCC2 being used.

Step12. AMF sends the NGAP PATH SWITCH ACK message back to gNB3, including a new pair {NH3, NCC3}.

### 3.X.3	Evaluation

TBD

*** End of 2nd Change ***