3GPP TSG-SA3 Meeting #117 	S3-242999

# Maastricht, NL, 19 - 23 August 2024

# Source:	Apple

# Title:	New solution to address key issue#1

# Document for:	Approval

Agenda Item:	5.6

# 1	Decision/action requested

Approve the pCR to TR 33.749

# References

- TR 33.749 v0.2.0

- TS 23.558 v19.2.0

- TS 33.558 v18.2.0

# 3	Rationale

This contribution proposes a new solution to address key issue#1 in TR 33.749.

# 4	Detailed proposal

*** Start of 1st Change ***

## 6.X	Solution #X: EEC proviced information verification

### 6.X.1	Introduction

This solution addresses key issue #1: "5G system should support a mechanism to verify the user information provided by EEC".

Clause 8.6.5 of 3GPP TS 23.558 [3] defines UE identifier API which is used by an EAS or EEC to obtain the identifier of the UE if the EAS or EEC does not have it (e.g. it has not already cached).

When EEC invokes the API, if available, this IE contains both UE’s private IPv6 address (due to the existence of NAT66) and UE’s private IPv4 address. According to TS 33.558, EEC authentication is not mandatory, there might be cases that EEC is not authenticated. When EEC is not authenticated, there is risk that the information provided by EEC is not owned by this EEC, leading to the result that the UE indentifier API is abused and other UE information is leaked.

This solution proposed to add the varification method when EEC authentication is not performed.

### 6.X.2	Details

Preassumption: the authenticated EEC will not provide untrusted user information.

Following the procedure in TS 23.558 on EEC requesting UE identifier, it shall provide the security credentials. It is proposed in this solution, that the security credential in the UE Identifier API request can be a certificate to EES to verify, when the EEC doesn’t support any other authentication methods defined in TS 33.558.

Step 1. EEC sends the UE Identifier API request, including the User Information (i.e.UE address), EEC signs the User Information using its private key. EEC also includes the Cert information as the security credential field as defined in TS 23.558[3].

Step 2. EES verifies the certificate. If the verification is successful, EES verifies the signature of the User information using the public key.

Step 3-6. Following the TS 23.558 clause 8.6.5.2 step 2-4, EES retrieves UE ID fro NEF and sends it back to EEC.

### 6.X.3	Evaluation

TBD

*** End of 2nd Change ***