3GPP TSG-SA3 Meeting #117	S3-243002

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Nokia

# Title:	Solution 6 update with evaluation

# Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

Approve the solution 6 update added to TR 33.700-29.

# 2	References

[1]	3GPP TR 33.700-29 Study on Security Aspects of 5G Satellite Access in the 5G Architecture phase 3

[2]      3GPP TR 23.700-29 Study on integration of satellite components in the 5G architecture

# 3	Rationale

The contribution proposes an update of the solution for MME and also evaluation to existing solution#6 of TR 33.700-29 [1] for primary authentication and NAS security context establishment during store and forward operations for Editor note removal.

Editor’s Note: Further evaluation is FFS.

# 4	Detailed proposals

# * * * * First change* * * *

## 6.6	Solution #6: Primary authentication and NAS security context establishment during store-and-forward operations

### 6.6.1	Introduction

This solution proposes the following:

- Including Satellite Access Type information in Initial UE message from Satellite gNB to AMF, to allow serving network AMF know about possible intermittent loss of feeder link communications.

- Updating NAS timers (e.g. T3510, T3511) to enable primary authentication and NAS security context establishment with UEs during store-and-forward operations as per legacy procedures.

### 6.6.2	Solution details

Figure 6.6.2-1: Message flow for security procedures between UE and NAS with S&F operations

Figure 6.6.2-1 shows the message flow for secure primary authentication and NAS security context establishment in S&F operations. In this solution:

- The delays incurred due to intermittent loss of feeder link connectivity can be minimized by using ISLs to make the NAS Registration Request message reach the ground network. However, since SA2 assumes that Store and Forward Satellite Operation shall work without ISL, this optimization is not considered in this solution.

- If timer T3510 expires at AMF or at UE, a new Reject cause is proposed which indicates that the rejection is due to delays in S&F operations. Also, the reject message with this new cause includes updated NAS timers for S&F operations.

  - Updated values of NAS timers can be computed, for e.g., by observing the time between first and second attempts of NAS Registration Request from UE.

  - Also, the satellite which does not have feeder link connectivity when it receives the RRC Connection Setup Complete can provide an estimate of the time needed for the round-trip communications between itself and core network for primary authentication and NAS Registration procedures.

- If the NAS timers for S&F operations are pre-computed, SoR or UPU or OTA methods maybe used to update the NAS timers in UEs which connect via the satellite access.

- As the Satellites are moving, it is likely that timer values vary for different UEs in the same coverage area. Considering this fact, the AMF can store the computed value and statistically calculate more accurate NAS timers for SAT access.

- In multi-satellite scenarios, especially considering LEO satellites which may move out between two consecutive NAS registration attempts from the UEs, AMF and core network can independently compute the required NAS timer values for satellite access type using information about satellite movements, as well as statistically analysing the time between two consecutive NAS registration attempts received by the core network from the same UE.

  - NOTE: This solution focuses on the timers associated with UE authentication and NAS registration procedures and does not consider handovers between satellites (when UEs are in RRC Connected mode).

MME architecture: Same solution will work for the MME architecture as well with dynamic adaptation of timer in Attach reject message.

### 6.6.3	Evaluation

Impacted entities:  UE, RAN and AMF

This solution enables re-use of legacy procedures for UE authentication and NAS security context establishments with satellite access type in store-and-forward operations by adapting NAS timers according to the delays observed between the UEs and the core network.

•            Assumptions: This solution assumes the complete gNB/eNB is in the Satellite. AMF/MME is not part of satellite, but solution works in those cases also with complete AMF/MME on board or split architecture also.

•            Dependency on SA2 or RAN: This solution is in line with timer details mentioned in the conclusion 8.2 of TR 23.700-29[2].

•            Relevant KI and Potential Security Requirements addressed: KI#1, Requirements#1.

•            Architecture option: At least, RAN onboard the satellite.

•            Re-use of legacy security procedures: Complete re-use of AKA procedure as this solution adapts the new timer values for AKA procedure.

•            Normative work expected for this solution: For adaptation of NAS registration reject message and timer adaptation to be given as input to SA2/CT.

•            Advantages of the solution: Dynamic adaptation of NAS timer will optimize and sync the Store and procedure Satellite with legacy UE AKA procedure.

•            Disadvantages of the solution: NA

•            Impacted entities: No impacts from SA3 point of view, however MME/AMF and UE (for NAS timer Registration reject message) are impacted.

NOTE: Unprotected reject message is not specific to satellite access and store-and-forward scenarios. Hence, this aspect is not considered in this solution.

# * * * * End of changes * * * *