3GPP TSG-SA3 Meeting #117	S3-243007

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Nokia

# Title:	EN removal for solution #4

# Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

Approve the EN removals for solution #4 in TR 33.794

# 2	References

[1]	TR 33.794, 'Study on enablers for Zero Trust Security', (Release 19)

# 3	Rationale

The contribution proposes changes to existing solution#4 for removal of following ENs.

Editor’s Note: If DCCF or NWDAF is re-used, the security risk of handling security data together with non-security related data needs to be evaluated against best practice of handling security logs, e.g. in NIST SP 800-92.

Editor's Note: Authorization of such security services by NFs is FFS.

# 4	Detailed proposals

*****Start of Change 1*****

## 7.4	Solution #4: Security data collection and exposure to enable detection of compromised NFs in SBA layer

### 7.4.1	Introduction

Figure 7.4.1-1: High level view of proposed solution

NOTE: In Figure 7.4.1-1, the NF security data collection agent/function is a logical function. Data collection and coordination function (DCCF) is already defined and can be reused here. Handling security data together with non-security related data needs to be evaluated against best practice of handling security logs, e.g. in NIST SP 800-92. Therefore, re-use of DCCF or NWDAF can be avoided, and, this security data collection functionality may be implemented as a separate entity or as a NF. This should be considered during implementation of operator’s security monitoring and evaluation system.

This solution proposal focuses on periodically collecting counters because:

- Counters are light weight from performance point of view.

- Periodically collecting counters allows feeding structured data to any security monitoring and evaluation functionality.

- Patterns can be derived from collection of such counters to allow obtaining better classification between NF misbehaviours due to SW bugs and actual security concerns in the system.

This solution proposes collection of one or more of specific security counters to assess if any NF is compromised in the SBA layer. Dynamic policy enforcement can allow operators to perform such data collection from selected set of NFs depending on the indicators of any malicious activity or potentially compromised NFs. Operator can configure the relevant security policies which define the thresholds and other parameters which may be required for collecting counters relevant for monitoring and detecting malicious activity of other NFs.

The security data proposed to be collected in this solution can be optional depending upon operator’s policies. This solution does not mandate collection of all the security data proposed here. Implementations may vary in terms of collecting all of this data, or a subset of this data, or some additional data as required as per operator’s policy configurations.

Following security data is proposed to be collected at periodic observation/monitoring intervals.

OAM Data:

- Audit logs like defined in TS 33.117

- Counters related to number of un-authorized attempts to access NFs.

- Counters related to authentication failures for obtaining access to NFs.

SBA layer data:

- Subscription and notification based security data. For example, security data collection agent/function can subscribe to different NFs for obtaining security events related to number of un-authorized/unauthenticated attempts to access the respective NFs.

Such subscription based security data can provide the flexibility of obtaining data from selected set of NFs during runtime, and, the subscriptions can be based on a targeted analytics being performed by Operator’s security monitoring and evaluation function  (or NWDAF if re-used).

When security data collection agent/function subscribes to NFs for receiving security events and data from NFs, NFs can ensure that only an authorized function/service is able to subscribe to receive security data.

*****End of Change 1*****