3GPP TSG-SA3 Meeting #117	S3-243008

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Nokia

# Title:	Evaluation for solution #4

# Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

Approve the solution evaluation for solution #4 in TR 33.794.

# 2	References

[1]	TR 33.794, 'Study on enablers for Zero Trust Security', (Release 19)

# 3	Rationale

The contribution proposes an evaluation to existing solution#4.

# 4	Detailed proposals

*****Start of Change 1*****

### 7.4.3		Solution Evaluation

Counters related to authentication and authorization failures can enable detection of malicious NFs in the system. Maintaining historic data of such counters can enable statistical analysis and detection of anomalies in the system. Moreover, collecting counters are lightweight and have minimal impact on the performance of the system. Also, audit logs and subscription based security event data obtained from NFs can be used to correlate with the counters and perform offline analysis to detect possibly compromised NFs in the system.

Following can be considered for normative work based on this solution proposal:

- Collection of counters related to number of un-authorized attempts to access NFs.

- Collection of counters related to number of authentication failures for obtaining access to NFs.

- Collection of audit logs

- Subscription based notifications from NFs for security events related to of un-authorized/unauthenticated attempts to access the respective NFs.

*****End of Change 1*****