3GPP TSG-SA3 Meeting #117	S3-243036

Maastricht, Netherlands 19 - 23 August 2024

# Source:	Huawei, HiSilicon

# Title:	User ID privacy protection based on EAP-TLS protocol using pseudonym mechanism

# Document for:	Approval

Agenda Item:	5.10

1	Decision/action requested

Approve the pCR to TR 33.700-32

2	References

[1]	3GPP TR 33.700-32, ' Study on security aspects of User Identities and Authentication’.

3	Rationale

This contribution proposes a solution to address the key issue #2 in TR 33.700-32 [1].

4	Detailed proposal

*** Start of 1st Change ***

6.Y	Solution #Y: User ID privacy protection based on EAP-TLS protocol using pseudonym mechanism

6.Y.1	Introduction

This solution addresses the key issue #2. The user’s privacy protection is based on a pseudonym mechanism to protect the User Identifier (ID). The user ID is replaced with a Member ID (MID) and a different MID is computed each time; hence increases the anonymity and avoid a user ID being tracked. In this solution, the EAP-TLS protocol is used to illustrate how the MID can replace the user ID to achive anonymity.

6.Y.1	Solution details

Aiming at the problem of identity information leakage during the authentication procedure, the solution introduces a new pseudonym mechanism. Users’IDs are grouped in advance and assigned Group User ID (GID GUser ID) to the UE. In the identity authentication phase, the EAP-TLS uses Member ID (MID) to replace the unique identity of the user and sent MID to the authentication server (AAA). In order to preserve the privacy of the unique identifier of the user, the AAA utilizes the GID to determine the users’ IDs group and the MID to determine the user's identifier.

Figure 6.Y.1 Illustration of user ID privacy protection based on pseudonym mechanism.

0. Provisioning: before the user authentication procedure, the authentication server (e.g AAA) and the UE are provisioned with the Group User ID (GIDGUser ID)

1. After Primary authentication, the UE sends to the SMF a User Activation Request message including the GID, a random number NID, the Member ID (MIDMUser ). The UE usesNUser ID  the user ID, NID and GID as intputNUser ID  to perform a hash operation to obtain the Member ID (MIDMUser ).

NOTE: The purpose of the random number NID NUser IDis to ensure the UE calculates a different MID MUser each time; hence it increases the anonymity and avoids User ID being tracked or linked to previous authentication.

2. The SMF initiates the user authenitication and sends to AAA the Authentication Request, including MIDMUser , NID NUser IDand GID.  GUser ID

3. The AAA GUser ID determines the users’ IDs group GID, finds the User ID k User IDk of each user in the group, and the AAA calculates MID K MUser Kwith the NIDNUser ID, User ID k and GIDGUser ID. Then AAA compares whether MID MUser  and MID K  MUser Kare equal. If they are equal, the AAA performs the subsequent user identity authentication and authorization based on the user ID. Otherwise, a failure message is gnerated.

4. After the EAP authentication completes, the AAA sends an EAP-Success message to the SMF, includingGUser ID  authentication/authorization result.

5. Based on the result of user authentication (Success/Failure), the SMF returns User Activation Response to the UE.

6.Y.3	Evaluation

This solution addresses the key issue #2.

The solution provides user privacy by using a Member ID (MID) instead of the real user identifier. Since the user identifier is not in the realm of 3GPP systems, how user identifier is mapped to MID is transparent to the 3GPP systems. To prevent tracking or linking of the user, the MID may occasionally be updated to the 3GPP system.

*** End of 1st Change***