3GPP TSG-SA3 Meeting #117	S3-243044

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Ericsson

# Title:	Update of Solution #5: UE authentication and traffic protection in ATSSS-Lite

# Document for:	Approval

Agenda Item:	5.15

# 1	Decision/action requested

It is proposed to approve the following updates to Solution #5 of [1].

# 2	References

[1]	TR 33.754

# 3	Rationale

This document proposes some updates to Solution #5 of TR 33.754 [1], mainly to resolve Editor's notes.

The first Editor's note in 6.5.2.2 states:

Editor's Note: Whether MPQUIC connection is established over 3GPP access or non-3GPP access is FFS.

In solution #5 it is clearly specified that MPQUIC connection is established over the 3GPP access. Solution #5 relies on the fact that the UE authenticates and registers over 3gpp access and that the MA PDU and MPQUIC are both established over that access. The non-3gpp access is later added as an additional MPQUIC path. It is thus proposed to remove the first Editor's note without further changes.

The second Editor's note in 6.5.2.2 states:

Editor's Note: Whether server-side only authentication is enough is FFS.

Only a UE authenticated by the 5GC can setup a PDU session via SMF. This is not unique for ATSSS, but this is true for all PDU session establishments. Since the MPQUIC connection is established over the PDU session, there is no need for further authentication of the UE. It is proposed to remove the Editor's note in the solution but add text in the Evaluation clause to keep it in mind for the conclusion phase where the different solutions are compared.

The Editor's note in 6.5.2.3 states

Editor's Note: Certificate based authentication or PSK based authentication is FFS and how the credential is provided is FFS.

In solution #5 it is clearly specified that server-side authentication is used when MPQUIC connection is established. In practice that always means using certificate-based authentication, but some clarifying text about this is proposed to be added in order to remove the E.N. The added text is however proposed to be placed in 6.5.2.2 and not where the E.N was placed.

# 4	Detailed proposal

*** START OF CHANGES ***

## 6.5	Solution #5: UE authentication and traffic protection in ATSSS-Lite

### 6.5.1	Introduction

This solution addresses key issues #1 and #2.

### 6.5.2	Solution details

#### 6.5.2.1 	Background

There are several potential architectures provided in the TR 23.700-54 [2]. This solution aims at performing an analysis of the architecture proposed in Solution #2.8 of TR 23.700-54 [2] in relation to the security requirements in KI#1 and KI#2.

Solution #2.8 of TR 23.700-54 [2] is based on the architecture displayed in Figure 6.5.2-1 (copied from [2]), where there is no N3IWF/TNGF used between UE and UPF. There is no N1 connection and hence no NAS connection between UE and 5GC over non-3GPP access.

The solution assumes the use of a Multi access PDU (MA PDU) session using MPQUIC [8]. Further, the solution assumes that the MA PDU session is established over 3GPP access. After that the UE can get user plane resources over non-3GPP access by adding a QUIC path that will run over non-3GPP access.

Figure 6.5.2.1-1: Architecture for simplified ATSSS over non-3GPP based on direct MPQUIC connection between UE and UPF as described in Solution #2.8 of 23.700-54.

#### 6.5.2.2 	UE authentication and PDU establishment over 3GPP access

UE authenticates and registers to the 5GC using existing procedures. Once registered, the UE continues to setup a MA PDU session with the selected UPF. The SMF selects a UPF that supports this type of ATSSS. How this is performed is described in the two options of Solution #2.8 of [2] but has no security impact.  The UPF also receives an indication for "direct ATSSS via non-3GPP access using MPQUIC".

The UPF allocates MPQUIC proxy information for both the N3 tunnel used via 3GPP access and for the Nx interface (non-3GPP access), i.e. the UPF allocates separate IP addresses and ports of the MPQUIC proxy in UPF for N3 and Nx interfaces (server side IP addresses and ports). The UPF also allocates the IP address for the UE to be used over the 3GPP access but not an address for non-3GPP access. (The UE for its non-3GPP access gets its IP address from the non-3GPP access.)

The information generated by the UPF is transferred to the UE via SMF-AMF. The information is protected over N1 using NAS security.

The UE then establishes a Multipath QUIC connection over the 3GPP access. The UE can also choose to setup multiple QUIC connections, one for each QoS flow, but for the rest of this analysis, we assume only one Multipath QUIC connection to be used with 2 paths, one for 3GPP and one for non-3GPP.

The QUIC connection is secured using TLS 1.3. During the establishment, the UE authenticates the UPF (server side only authentication). Hence, the UE is not explicitly authenticated by the UPF, and this is not needed either, since only a UE authenticated by the 5GC can setup a PDU session via SMF. This is not unique for ATSSS, but this is true for all PDU session establishment.

Note that it is not necessary for the client (UE) to authenticate the server (UPF) either in the context of ATSSS, for the same reason as above. TLS however mandates the server to be authenticated, for example by sending its certificate to the client, but it is it is not mandated for the client to verify the certificate using a root certificate. It could for example be a self-signed certificate.

Hence, the UPF needs to be configured with a server certificate and the UE can optionally be configured with the means to verify that certificate.

For each QUIC connection the UE obtains the following information from the UPF using inherent QUIC mechanisms:

-	At least two Connection IDs for a QUIC connection: A QUIC connection can be associated with multiple Connection IDs. To support multi-path QUIC operation, the QUIC endpoints shall use different Connection IDs on different paths (see [8]). In this case, a minimum of two separate Connection IDs will be used, one for the 3GPP access and one for the non-3GPP access.

#### 6.5.2.3 	UE authentication over non-3GPP access

Once a MPQUIC connection is established via 3GPP access, the UE can add non-3GPP access user plane resources.

Figure 6.5.2.3-1 (copied from [2]) shows the procedures for adding a path over non-3GPP access.

1.	UE has an established MPQUIC connection with the UPF over 3GPP access. During the setup of the MPQUIC connection, at least 2 Connection IDs were allocated, one used over 3GPP access and the other to be used over non-3GPP access.

2.	UE obtains a local IP address from the non-3GPP access.

3.	UE initiates the path validation of the new path with the UPF via non-3GPP access as defined in the QUIC specification [6] and the QUIC multi-path extensions ([8]). The path validation enables the UPF to verify the IP address of the UE.

The path validation is performed using the same QUIC connection and the same security context as being used for the path over 3GPP access. Hence there is no explicit authentication of the UE over non-3GPP access. From the UPF perspective this is the same UE using the same QUIC connection over an added path. The UPF is not aware of the underlying access type.

4-6.	As specified in solution #2.8 of [2]

7.	The UE and UPF can start sending data packets via the new path. The data is confidentiality and integrity protected. The credentials are the same as for the first path since both belong to the same QUIC connection.

Solution #2.8 of TR 23.700-54 [2] further specifies that the UPF shall block incoming traffic to the MPQUIC proxy address for non-3GPP access that is not associated to existing QUIC connections. This ensures that only UEs with valid QUIC connections can communicate with the UPF via non-3GPP access.

Figure 6.5.2.3-1: Addition of non-3GPP access user-plane resources

### 6.5.3	Evaluation

#### 6.5.3.1 	UE authentication (KI#1)

The UE is authenticated over 3gpp access using existing primary authentication procedures. The authenticated 3gpp access is used for setting up the MPQUIC (TLS) connection. The MPQUIC connection is established using server-side authentication based on certificates. Hence, the UE is not explicitly authenticated by the UPF, and this is not needed either, since only a UE authenticated by the 5GC can setup a PDU session via SMF and the MPQUIC connection is established over the PDU session. This is not unique for ATSSS, but this is true for all PDU session establishments.

The same MPQUIC connection and hence the same security context is used for the non-3gpp access. UPF does not see this as a new access, simply a new path using the same security context.

#### 6.5.3.2 	Confidentiality and integrity protection (KI#2)

Both paths apply confidentiality and integrity protection using credentials established during the setup of the QUIC connection which is using TLS 1.3 [9].

#### 6.5.3.3	 Privacy considerations (KI#1)

A fresh Connection ID is used for each new path (or when migrating from one path to another) as defined in the QUIC RFC [6]. The Connection IDs are pre-allocated by the server (UPF) during the QUIC connection establishment. Usage of fresh Connection IDs prevents the linking of different paths to one another.

#### 6.5.3.4 	System impact

Impacted entities are:

- 	UE

- 	AMF/SMF (depending on option 1 or option 2 of Solution #2.8 of TR 23.700-54 [2])

- 	UPF

*** END OF CHANGES ***