3GPP TSG-SA3 Meeting #117	S3-243145

Maastricht, Netherland, 19th - 23th August 2024

Title:	[DRAFT] Reply LS on User Identities and Authentication Architecture

Response to:	LS S2-2407236 on User Identities and Authentication Architecture from SA2

Release:	Rel-19

Work Item:	Study on User Identities and Authentication Architecture (FS_UIA_ARC)

Source:	Ericsson, to be SA3#117

To:	SA2

Cc:	SA3-LI, SA1

Contact person:	Ferhat Karakoc

Ferhat.karakoc@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA3 would like to thank SA2 for the LS S2-2407236 on User Identities and Authentication Architecture.

Regarding the questions from SA2, SA3 has the following replies.

Question1a from SA2:

During the discussion related to Key Issue #1 and Key Issue #2 most companies believe that it is appropriate to send the user identifier in NAS (e.g., to trigger authentication by the network and to activate the user with a subscription). Different from the majority view, some companies objected to this conclusion and raised the issue of trust of the user identifier and credentials for the MNO (HPLMN) and questioning whether the HPLMN will be able to trust the user identifier and credentials.

While considering that the views from majority of companies being,

- the user identity and credentials that will be used to identify the user in HPLMN might not be stored in UICC. For example, a User Interface could be used to retrieve user identifier and credentials directly from the human user; or the User Interface could identify the human (e.g., based on biometric authentication) and related human user identifier and credentials could be retrieved by the operating system (e.g., from the UICC or other local storage).

- procedures and methods similar to NSSAA will be used to authenticate the user.

SA2 requests SA3 to provide feedback if their views are inline to the majority view as described above.

Reply to Question 1a from SA3:

SA3 sees the following issue with using a NAS-based procedure (e.g. similar to NSSAA) for user authentication: NAS-based procedures for user authentication require an interface between user and modem. However, such an interface does not exist in current devices. Furthermore, they open up the NAS layer for user input. This can lead to several security issues, e.g. DDoS attacks on 3GPP functions from misbehaving applications on the UE.

Question1b:

Regarding the Key Issue #3 of TR 23.700-32, can any existing information of the UE subscription with whom the user identifier is active, or new information related to the user to be allowed to be exposed to an authorized AF, and if so, is user consent required for the exposure of this information? Specifically, we would like to understand the user consent requirements for the following new information:

- the verification result indicating whether the User Identifier authentication is successful.

Reply to Question 1b from SA3:

Whether user related information can be exposed and whether additional security measures are required, depends on the kind of information that is to be exposed. SA3 would require a clearer description of the data to be exposed to answer the question.

Regarding the verification result indicating whether the user identifier authentication is successful, SA3 would like to point out that the verification result is not enough for possible use cases that require verification that a specific user is present in certain communication. Such verification is only possible using key establishment that binds the authentication to further communication e.g. authorization of financial transactions or sharing of information.

Question2:

Regarding Key Issue #1 of TR 23.700-32, SA2 kindly requests SA3 to provide their opinion whether there is a privacy issue if the network keeps providing SMS and IMS services to a UE when a user identifier is active with the UE subscription.

Reply to Question 2 from SA3:

There is a privacy issue, SMS and IMS services of the UE subscription should not be offered to a user using a UID if the user using the UID is not authorized to see or get SMS/IMS terminating services. However, the authorization can be handled by authorization of applications at the UE itself.

# 2	Actions

To SA WG2

ACTION:  SA3 kindly asks SA2 to take the above answers into account.

# 3	Dates of next TSG SA WG 3 meetings

SA3#118	14 - 18 October 2024		Hyderabad (India)

SA3#119	11 - 15 November 2024		Orlando (US)