3GPP TSG-SA3 Meeting #117	S3-243232

Maastricht, Netherlands  19 - 23 August 2024

Title:	Reply LS on enhancement to the protocol stack of IMS Data Channel

Response to:	LS S3-242427/S4-241373 on enhancement to the protocol stack of IMS Data Channel from SA4

Release:	Rel-18

Work Item:	5G_MEDIA_MTSI_ext

Source:	China Mobile (to be SA3)

To:	SA4

Cc:	SA3-LI

Contact person:	Xiaoting Huang

huangxiaoting@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

SA3 thanks SA4 for the LS on enhancement to the protocol stack of IMS Data Channel. SA3 provides the following response:

Q1: Would SA3 see any requirement of mandating the usage of IMS media plane encryption?

A1: In 33.328 clause 5.1, it has been stated that “the support for IMS media plane security mechanisms and procedures is optional in IMS UEs and its support in the IMS core network is also optional. The support of IMS data channel media is optional. ”Thus SA3 confirms that the usage of IMS media plane encryption is not mandated.

Q2: Could you confirm that the MNO can choose whether to encrypt IMS audio and video media during the media negotiation with UE, in IMS?

A2：TS 33.328 specifies the security association set-up procedures for media protection in clause 7, where MNO is able to control whether to encrypt IMS audio and video media. In details, when performing the registration, an IMS UE supporting the mechanisms required for e2ae protection includes an indication "e2ae-security supported by UE" in the initial REGISTER message, when the P-CSCF (IMS-ALG) is capable of supporting the mechanisms required for e2ae protection according to this specification, and the network policy is to prefer e2ae protection for this registration, the P-CSCF (IMS-ALG) includes an indication "e2ae-security supported by network" in a message to the IMS UE during registration. That means, the MNO controls whether to encrypt IMS audio and video media in the configuration of network policy of e2ae protection.

Q3: Could the MNO choose whether to encrypt DC media during the media negotiation with UE, similar to how they can choose to encrypt IMS audio and video media?

A3: Same as above, the MNO can choose to encrypt DC media or not by the indication of “e2DCe-security for IMS data channel supported by the network”.

2	Actions

To SA4

ACTION: 	SA3 kindly asks SA4 to take the answer into account when determining protocol stack enhancements.

3	Dates of next TSG SA WG 3 meetings

SA3#118	14 - 18 October 2024		Hyderabad (India)

SA3#119	11 - 15 November 2024		Orlando (US)