3GPP TSG-SA3 Meeting #117	S3-243249

Maastrict, NL, August 19 – 23, 2024

# Source:	OPPO

# Title:	Discussion on security handling for inter-CU LTM in non-DC cases

# Document for:	Discussion

Agenda Item:	4.12

1	Decision/action requested

It is proposed to discuss and take a decision on RAN2’s options.

2	References

[1]	LS R2-2404037

[2]	3GPP TS 38.300

3	Rationale

3.1	Overview of R19 inter-CU LTM

According to RAN2 LS R2-2404037 [1], the inter-CU LTM will be introduced in R19. As the following figure illustrates, the inter-CU LTM differs from the intra-CU LTM in that it involves a UE switching between different CUs, and security key update is needed.

As specified in clause 9.2.3.5 of TS 38.300[2], LTM (L1/L2 Triggered Mobility) is a procedure in which a gNB receives L1 measurement report(s) from a UE, and on their basis the gNB changes UE serving cell by a cell switch command signalled via a MAC CE. The cell switch command indicates an LTM candidate configuration that the gNB previously prepared and provided to the UE through RRC signalling. Then the UE switches to the target configuration according to the cell switch command.

Figure 3.1: Difference between intra-CU LTM and inter-CU LTM

In the LS R2-2404037, RAN2 has discussed the aspect of inter-CU LTM with security key change and RAN2 have viewed some options as possible directions for handling the key change as part of inter-CU LTM cell switch. Hence those options should be analysed from security perspective to give RAN2 feedback.

3.2	Analysis of RAN2’s Options

Option 1: Use a new information in MAC CE to deliver the security information. Whether the UE uses horizontal or vertical key derivation is derived from this new information in MAC CE (which is currently, neither integrity protected nor ciphered). Option 1A: the NCC value to be used at inter-CU LTM execution is included in the LTM cell switch command MAC CE. Option 1B: the UE is preconfigured with a list of NCC values in a ciphered and integrity protected RRC message and the index of an NCC value in the list is included in the LTM cell switch command MAC CE.

In Option 1, the NCC value or an index of NCC value is transferred between gNB and UE vie MAC CE, the MAC CE is neither integrity protected nor ciphered. However, even though the NCC value is not direct used in the input parameter of KNG-RAN* derivation, it is an essential parameter to indicate the UE whether to perform a vertical handover or a honrizantal handover, also how many times calculation should performed to derive the NH if a vertical handover happens.  Hence, a key mismatch may happen easily if unprotected MAC CE containing the NCC value or the index of NCC value is tampered with.

Observation 1.  The Option 1 is not acceptable from security perspective, unless the NCC value included in the MAC CE is protected

Option 2: Similar to Rel-18 S-CPAC key update mechanism, the UE is preconfigured from the source gNB with a list of NCC values per CU using RRC signalling (that is both integrity protected and ciphered). It is expected that the participating gNBs (CUs) would need to be aware of the list and how the UE applies the list during LTM cell switches: Option 2A: UE chooses the first unused NCC for the target CU upon inter-CU LTM execution. Option 2B: As an alternative to choosing the next unused NCC (as in option 2A), horizontal key derivation is used in this option if the LTM cell switch is between the same two CUs.

In Option 2, a list of NCC value is preconfigured in UE per CU by the source gNB. The following figure shows one potential configuration method of the NCC values. If the UE choose an unused NCC value, it means a vertical handover performs. Considering the NH is derived from KAMF which can only be generated by UE and AMF, the NH should be configured to candidate gNBs per CU as well. And the candidate gNB(s) should be configured with {NH, NCC} pairs by the AMF.

The out-of-oder utilization of NCC value and corresponding NH may easily occur, e.g., UE switch from CU#a to CU#c and then swich to CU#b. And it should also be taken into consideration, that the NCC has only eight possible values according to TS 24.501, due to the potential depletion of NCC values after repeated inter-CU handovers, and the shared usage of NCC values for L3-based vertical handovers and inter-CU handovers, a scenario may arise where no available NCC value exists. To allocate available NCC value to the UE, and allocate available {NCC, NH} pairs to candidate gNBs, the KAMF should be updated, otherwise same NHs will be derived. A primary authentication may be triggered by the Network to update the KAMF.  Even RAN2 has not decided yet the maximum number of candidate cells in Rel-19, the running out of NCC may easily occur in the NCC pre-configured option, frequent primary authentication operation will cause a loss of network resources.

Figure 3.2: a potential NCC value configuration method in Option 2

Observation 2.  Due to the potential depletion of NCC values after several inter-CU handovers, and the shared usage of NCC values for L3-based vertical handovers and inter-CU handovers, a scenario may arise where no available NCC value exists. Therefore, some measures are needed to address the NCC running-out issue.

Option 3: After the execution of inter-CU LTM cell switch, the participating gNBs are expected to be updated with new K-gNB* to be used for the next inter-CU LTM cell switch. The UE and CN are aware of how the UE would use the next NCC value. Option 3A: The UE determines the following NCC value to use by itself (e.g., increase by 1) after subsequent inter-CU LTM execution. Option 3B: UE is pre-configured by the CN (via source gNB RRC signalling) with a list of NCC values and the UE chooses the first unused NCC value as the next NCC value.

In Option 3, the UE and CN are aware of how the UE would use the next NCC value, the target gNB should be aware of the NCC and NH as well. According to TS 33.501[3], as NH parameters are only computable by the UE and the AMF, it is arranged so that NH parameters are provided to gNB/ng-eNBs from the AMF in such a way that forward security can be achieved. If the piar of {NCC, NH} is received from the source gNB, the source gNB is able to know the target gNB keys, and the forward security can not be achieved. Hence, the target gNB should receive the NH from CN.

Similar with option2, the potential depletion of NCC values after repeated inter-CU handovers should be considered. In Option 3A, the NCC value is increased by UE itself.  And in Option 3B, a list of NCC values are configured by CN, which is the main difference between the NCC configuration method of Option 2.

Observation 3. To achieve forward security, the NH parameter needs to be distributed from the AMF to the target gNB. And the source gNB can not access the NH parameter belonging to the target gNB.

Option 4: After every inter-CU LTM cell switch execution, the UE is provided via RRC signalling with the NCC value to be used by the UE for key derivation at the next inter-CU LTM cell switch.

In Option 4, there is a new RRC message from the serving gNB, which carries the NCC value to provide to UE for the next inter-CU LTM security updating. While, the candidate gNB (s) should also be configured with the {NCC, NH} pair. This option is not complete, and may cause a situation that the NCC value provision to the UE is a litter bit early, since the Kamf may change before the UE next inter-CU LTM cell switch.

Observation 4. The Option4 is not complete, from security perspective the potatial vulnerability of NCC transmission through unprotected connection can be avoided. The NCC pre-configuraition for the next inter-CU LTM may be a litter bit early, since the KAMF may be changed before the UE next inter-CU LTM cell switch.

3.3	Conclusion

According to the above analysis of all these options in LS R2-2404037, it is conclused that:

- Conclusion 1: The Option 1 is not acceptable from security perspective, unless the NCC value included in the MAC CE is protected.

- Conclusion 2: For both Option 2 and 3, the potential depletion of NCC values after several inter-CU handovers and L3 based handover should be taken into consideration. To achieve forward security, the NH parameter needs to be distributed from the AMF to the target gNB. And the source gNB can not access the NH parameter belonging to the target gNB.

The pre-allocation of vertical handover key materials may result the following diaadvantages: (1) Out-of-order using of the NCC; (2) Unused key materials need to be cleaned up; (3) If the KAMF changes, all {NH, NCC} pairs generated from the old KAMF need to be discarded, and new key metetial allocation is needed. To sum up, both option 2 and 3 may bring more impacts to the gNB and CN (e.g., AMF alloates the {NH, NCC} pair), and require more management for key materials.

- Conclusion 3: The Option4 is not complete, from security perspective the potatial vulnerability of NCC transmission through unprotected connection can be avoided. The NCC preconfiguraition for the next inter-CU LTM may be a litter bit early, since the Kamf may change before the UE next inter-CU LTM cell switch.

4	Detailed proposal

Proposal 1. Based on above analysis and conclusions, we propose that SA3 filters available options from RAN2’s options and discard some of options to narrow down the scope for the further study.

Proposal 2. It is proposed that SA3 works on the option 1 or option 4 for the further inter-CU LTM security standarlization.