3GPP TSG-SA3 Meeting #117	S3-243278

# Maastricht, Netherlands 19 - 23 Aug 2024

# Source:	OPPO

# Title:	Update KI#4 for Command

# Document for:	Approval

Agenda Item:	5.9

1	Decision/action requested

It is proposed to update KI on Protection of information during AIoT service communication.

2	References

[x]	 R2-2406150 “LS on RAN2 agreements and assumptions for Ambient IoT”

3	Rationale

It is proposed to update the KI#4 Protection of information during AIoT service communication in TR 33.713. Two kinds of AIoT services are going to be studied in R19, i.e., Inventory service, and Command service. This pcr is aimed to identify security threats and security requirements of Command service.

4	Detailed proposal

**** Start of Changes****

5.4	Key issue #4: Protection of information during AIoT service communication

5.4.1	Key issue details

As per TS 22.369 [2], Ambient power-enabled IoT (AIoT) services aim to support various use cases, including inventory taking, sensor data collection, asset tracking, and actuator control. These services intended to operate with lower power consumption and complexity than the existing IoT technologies such as eMTC, NB-IoT, and RedCap. To fulfil these requirements, AIoT devices require a communication capability.

TR 23.700-13[4] has a key issue pertains to support the AIoT services, i.e., Inventory and Command.  The following instructions of Command may be supported: reading data from AIoT device, writing data to AIoT device, disable the AIoT device temporarily or permanently. The AIoT device may response the Command with acknowledgement or optionally data (e.g. in case of the Read operation).

For RAN2 baseline, the “inventory and command” case is supported by the procedure[x]:

-	Step A: A-IoT paging;

-	Step B: Device ID transmission (via Random Access or without using RA).  Details are FFS

-	Step C: reader to device data transmission (e.g. the R2D command), and

-	Step D: corresponding device to reader data transmission (e.g. the feedback).  FFS whether this is optional, pending other WG discussions.

From a security perspective, security mechanisms to protect the information transmitted during AIoT service communication need to be supported. Failure to provide such security mechanisms will lead to various attacks such as eavesdropping, manipulation and/or unauthorized transmission of the information during AIoT service communication.

5.4.2	Security threats

Failure to protect the integrity of Command message will lead to attacks such as fabrication, modification, or removal of the instructions in the Command request.

If the Command message is not anti-replay protected, the Command message may be replayed by an attacker, and further cause exhaustion of AIoT device power.

Failure to protect the confidentiality of Command message will lead to attacts such as eavesdropping attacks.

5.4.3	Potential security requirements

The 5GS shall be able to support a means to support integrity and anti-replay protection of the Command message.

The 5GS shall be able to support a means to support confidentiality protection of the Command message.

**** End of 1st Change****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[x]	 R2-2406150 “LS on RAN2 agreements and assumptions for Ambient IoT”

**** End of Changes****