3GPP TSG-SA3 Meeting #117	S3-243280

Maastrict, NL, August 19 – 23, 2024

# Source:	OPPO

# Title:	New KI on AIoT Authentication of Command Use Case

# Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This document proposes an authentication KI on the command use case to TR 33.713.

# 2	References

[x]	 R2-2406150 “LS on RAN2 agreements and assumptions for Ambient IoT”

# 3	Rationale

The “inventory and command” case was agreed as RAN2 baseline [x]. Without authentication, command messages are not secured.

# 4	Detailed proposal

*** Start of Changes ***

## 5.X	Key Issue #X:  Key Issue on Authentication for AIoT system

### 5.X.1	Key issue details

This KI studies the authentication aspect in AIoT services. Without authentication, command messages are not secured.

For RAN2 baseline, the “inventory and command” case is supported by the procedure[x]:

-	Step A: A-IoT paging;

-	Step B: Device ID transmission (via Random Access or without using RA).  Details are FFS

-	Step C: reader to device data transmission (e.g. the R2D command), and

-	Step D: corresponding device to reader data transmission (e.g. the feedback).  FFS whether this is optional, pending other WG discussions.

Step C and Step D are command messages between AIoT device and network. From security point of view, before step C and step D, authentication is needed to prevent impersonate date transmission to AIoT device or network endpoint to mitigate potential impersonate threats.

### 5.X.2	Security threats

If no authentication before command messages[x], there is potential impersonate threat for AIoT service.

If authentication of network is not performed before step C, there is potential impersonate threat for the “reader to device data transmission (e.g. the R2D command)”, due to lack of way for AIoT device to make sure the “reader to device data transmission” is from the legitimate reader.

If authentication of AIoT device is not performed before step D, there is potential impersonate threat for the “device to reader data transmission (e.g. the feedback)”, due to lack of way for reader to make sure the “device to reader data transmission” is from the legitimate AIoT device.

### 5.X.3	Potential security requirements

The 5GS shall be able to provide a means to support authentication of network termination point before command message in step C.

The 5GS may be able to provide a means to support authentication of AIoT device before command message in step D.

Editor’s Note: The authentication requirements of AIoT device is optional, since in RAN2’s baseline step D is FFS whether this is optional.

*** End of 1st Change ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[x]	 R2-2406150 “LS on RAN2 agreements and assumptions for Ambient IoT”

*** End of 2nd Change ***