3GPP TSG-SA3 Meeting #117	S3-243349

Maastricht, Netherlands

19 - 23 August 2024

# Source:	Philips International B.V.

# Title:	Evaluation Solution #22

# Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

It is proposed to approve this pCR to update solution#22 in TR 33700-29-030.

# 2	References

[1]	3GPP TR 33.700-29-030

# 3	Rationale

Solution #22 in TR 33.700-29-030 does not include an evaluation. This Tdoc proposes an evaluation for said solution.

*** Start of Change ***

### 6.22.3	Evaluation

This solution addresses Key Issue #1 and applies to S&F operations in 5G.

The solution ensures that Feeder Link unavailability does not hinder the connection establishment between a UE and a serving SAT in S&F mode. The solution relies on a selective SAT Group/constellation allowing for reliable communication by a SAT, acting as the primary serving SAT to a UE and managing the coordination with other satellites, which maintains the continuity of operations and mitigates the risks associated with intermittent connectivity that is typical in S&F operations.

The solution, however, relies on ISL and thus, is not aligned with SA2’s architectural assumptions and principles indicating that S&F satellite operation shall work without ISL.

*** End of Change ***