3GPP TSG-SA3 Meeting #117	S3-243491-r3

Maastricht, Netherlands 19 - 23 August 2024

# Source:	Huawei, HiSilicon

# Title:	overall evaluation for KI#3

# Document for:	Approval

Agenda Item:	5.4

# 1	Decision/action requested

Approve to include pCR proposal for the TR 33.776

# 2	References

[1]	3GPP TR 23.776 	Study of Automatic Certificate Management Environment (ACME) for the Service Based Architecture (SBA)

# 3	Rationale

The contribution provides evaluation of all solutions addressing KI#3, i.e., Sol#1, Sol#2 and Sol#3, in TR 33.776 [1].

# 4	Detailed proposal

pCR

***  BEGINNING OF CHANGES ***

## 7	Conclusions

### 7.3	Overall evaluations for KI#3

There are three solutions addressing KI#3 on ACME challenge validation, i.e., Sol#1-Sol #3.

|  | IETF protocol adapted from, ACME identifier & Challenge Type |  | Impact to 3GPP or IETF (specs and deployment) |
| --- | --- | --- | --- |
| Sol#1 | RFC 8555 | ACME identifier= NF FQDN Challenge= http-01/dns-01 | Impact to 3GPP: -NF FQDN must be formed based on the NF instance ID. -a new interface is needed to modify DNS record for DNS challenge. |
| Sol#2 | RFC 9447 | ACME identifier= NF instance ID Challenge= tkauth-01 | To define* 1)new ACME identifier type “nf-instance-id”, and 2)procedure to specify the use of this identifier, tkauth-01 challenge and validation. Impact to 3GPP: -OAM: addtional role of a Token Authority; addtional interface with NF/ACME client with increased exposure surface and traffic load. -TS 33.310: addtional profile of ACME Authority Token |
| Sol#3 | RFC 8555 (*NOTE1) | ACME identifier= NF instance ID Challenge= NF instance ID | Impact to IETF: to define a new ACME identifier "NF instance ID", a new challenge type - "NF instance ID", and procedures for proof of NF control based on initial trust |

*NOTE1: A new challenge type needs to be defined in another specification other than RFC 8555.

Editor’s Note: the conclusions are FFS

For the solution #1, the NF instance ID and FQDN in an NF profile have to be linked so that the original ACME procedure RFC 8555 [2] can be reused. The related 3GPP specifications (e.g., SBA certificate profiles, NF profiles) have to be changed. Since the NF instance ID is widely used in SBA, the use case should be checked carefully so that the changes will not affect the SBA system running.

For the solution #2, the tkauth-01 challenge type is only defined for the use with “TNAuthList” identifier in IETF, a new ACME identifier “nf-instance-id” and how to use it with the challenge type tkauth-01 need to be defined.

Editor Note: whether can be done in 3GPP or IETF is ffs.

For the solution #3, a new ACME identifier "NF instance ID", a new challenge type for the "NF instance ID" identifier, and procedures for proof of NF control over the NF instance ID should be defined in IETF.

In general, all three solutions can achieve automated certificate validation based on/adapted from the ACME protocols. Different solutions may have different impacts to IETF/3GPP specification/deployment.

***   END OF CHANGES   ***