3GPP TSG-SA3 Meeting #117	S3-243537

Maastricht, the Netherlands, 19th - 23th August 2024

# Source:	ZTE Corporation

# Title:	Evaluation for solution #13

# Document for:	Approval

Agenda Item:	5.7

1	Decision/action requested

This contribution proposes evaluation for solution #13.

2	References

[1]	3GPP TR 23.700-29: "Study on integration of satellite components in the 5G architecture; Phase 3".

3	Rationale

This contribution proposes evaluation for solution #13 of TR 33.700-29 based on SA2 conclusion.

4	Detailed proposal

**** Start of Changes****

6.13	Solution #13: Security protection based on onboard HSS

6.13.1	Introduction

This solution address the key issue#1:Security protection in Store and Forward Satellite Operation.

6.13.2	Solution details

This solution based on the solution#18 in the TR 23700-29 [2] with following assumptions and principles:

-	The eNB, MME and HSS are placed on board the same satellite.

-	The UE has a subscription and credentials in the onboard HSS, the onboard HSS is synchronized with the HSS on the ground when feeder link is available.

-	Single satellite deployment use case, the UE accesses one satellite only which maintains a NAS and AS state of the UE.

-	No roaming support.

Given the above assumption, The authentication and NAS security can be accomplished when service link is available.

The onboard HSS is synchronized with the HSS on the ground when feeder link is available. It is assumed that a subset of UEs have their subscriptions and credentials in the onboard HSS. Different satellite stores distinct subscriptions and credentials for different subsets of UEs. If the UE accesses a satellite which has its subscription and credential, the UE can proceed to attach, otherwise the attach request is rejected due to the lack of UE subscription and credential.

6.13.3	Evaluation

Dependency on SA2 or RAN: Depends on the architecture option selected by TR 23.700-29 [2].

Relevant KI and Potential Security Requirements addressed: KI#1, Requirements 1, 2 and 3.

Architecture option: RAN and CN onboard the satellite.

Re-use of legacy security procedures: Full re-use of AKA procedure.

Normative work expected for this solution: No

Advantages of the solution:

- This solution reuses the legacy procedures for authentication and NAS security. No security enhancement is needed.

- There is no risk of Dos attack on the satellite since there will not be incomplete security procedures in this solution.

Disadvantages of the solution:

- Subscriber credentials are provisioned in several satellites HSSs. If one of the satellites is compromised by an attacker, either in the form that the attacker could obtain the subscriber credentials or that the attacker could control the interface to the HSS of the satellite, and if, for any given subscriber, the credentials in the HSS of the satellite were the same, this would imply that, for all subscribers whose credentials were stored in the compromised satellite, the subscriber credentials would have to be re-provisioned in the USIMs in the field and the satellites HSSs storing the subscriber credentials. Therefore, HSS on the ground needs to maintain a record of which UE credentials are in which satellite(s) at all times.

- The key used to generate authentication vectors should be stored on the satellite, which may not be the resources of the home operator.

Impacted entities: The on board HSS synchronizes the UE subscription and credentials with HSS on ground.

**** End of Changes****