3GPP TSG-SA3 Meeting #117	S3-243546

Maastricht, Netherlands 19 - 23 August 2024                                             revision of S3-243222

# Source:	Samsung

# Title:	Resolving EN in Sol#25

# Document for:	Approval

Agenda Item:	5.7

1	Decision/action requested

SA3 is kindly asked to approve this contribution.

2	References

[1]	3GPP TS 23.700-29 Study on integration of satellite components in the 5G architecture; Phase 3

3	Rationale

Solution#25 has the following Editor's Notes in Solution details.

Editor’s Note: How to determine the scope of UEs that need KSAT.UE is FFS.

Editor’s Note: Whether the permanent key should be stored in ME or USIM is FFS.

Editor’s Note: Whether SIB messages are only the source of freshness are FFS.

Editor’s Note: Whether step 7 needs to be protected is FFS.

For the first Editor's Note, similar to IOPS case, the satellite may be populated with KSAT.UE either for only the UEs that may access the satellite or all UEs that may access the satellite depending on the deployment and implementation.

For the second Editor's Note, as per the conclusion in TR 23.700-29 [1], UE may have a USIM dedicated to the satellite network.

For the third Editor's Note, there may be the source of freshness other than SIB message. It can be decided during normative phase.

For the last Editor's Note, unlike the protected message in step 2 due to the store and forward property, same security level as the ground network case can be achieved even if the message in step 7 is not protected, because the message in step 7 is processed immediately by the satellite which has Authetnciation Vector(s) to authenticate the UE.

4	Detailed proposal

*************** Start of 1st Change ****************

6.25	Solution #25: Solution on preventing DoS attacks before security context is established

6.25.1	Introduction

This solution addresses key issue #1: Security protection in Store and Forward Satellite Operation.

In store and forward operation, a satellite receives and stores the uplink data from UEs when only service link is available, and then the satellite forwards the data to the ground network when the feeder link becomes available. For example, the satellite may receive a number of registration request messages from malicious UEs, due to this store and forward property. The storage capacity of the satellite can be easily exhausted, which leads to DoS attack on the satellite. In S&F operation, moreover, UE is vulnerable to a fake satellite attack on the Attach Reject message as captured in some solutions in TR 23.700-29 [2].

In this solution, before the NAS/AS security context is established between the UE and network, key (KSAT.UE) derived from the permanent key is used for the authentication between UE and satellite, in order to prevent the DoS attack and false satellite attack. In this solution, it is assumed that a satellite includes eNB and the functionality of MME related to the authentication called MME (NT).

6.25.2	Solution details

Figure 6.25.2-1: Protected attach procedure with MME onboard the satellite in S&F operation

To prevent DoS attacks, the following procedure is preceded:

0. The satellite generates SAT.Nonce and provides it to HSS in GND. The HSS in GND derives KSAT.UE using the permanent key of the UE, ID of the eNB, and SAT.Nonce. The HSS provides KSAT.UE and IMSI to the satellite, and the satellite stores them with SAT.Nonce.

NOTE:	The permanent key may be uniquely assigned for S&F operation.

NOTE: The satellite may be populated with KSAT.UE either for only the UEs that may access the satellite or all UEs that may access the satellite depending on the deployment and implementation.

NOTE: Depending on deployment and implementation, UE may have a USIM dedicated to the satellite network.

After that, the authentication process of S&F mode is as follows.

PHASE 1. (Service link is available, Feeder link is unavailable)

1. The satellite provides a random number generated by the satellite (SAT.RN), S&F indicator indicating that the satellite is operating in S&F mode, and SAT.Nonce. These may be included in the SIB (System Information Broadcast) message.

NOTE: Whether other message can be used as the source of freshness is to be determined duinrg normative phase.

2. The UE derives KSAT.UE using the permanent key of the UE, ID of the satellite, and SAT.Nonce. The UE initiates the attach procedure by transmitting protected Attach Request message. This message consists of the Attach Request message (UE.RN, SAT.RN, and S&F indicator in addition to existing parameters) and MAC of the message, generated by the UE using KSAT.UE. S&F indicator indicates that the UE will operate in S&F mode.

The satellite checks the validity of the UE by verifying the MAC.

3. If the verification is successful, the satellite stores the received attach request message and transmits the protected Attach Reject message. This message consists of the Attach Reject message (Re-attach Info in addition to exsting parameters) and MAC of the message generated by the satellite using KSAT.UE. The Re-attach Info is information necessary for the UE to attempt the reconnection in step 7 (e.g., information about when the UE should retry to reconnect or list of satellite(s) that the UE should retry to reconnect).

The UE checks the validity of the satellite by verifying the MAC. If verified, UE waits for step 7 based on the guideline received.

PHASE 2. (Service link is unavailable, Feeder link is available)

4. The satellite requests authentication data for the UE by sending the Authentication Data Request message to the ground network (GND). The request includes IMSI, SN identity, and Network type.

5. Upon the receipt of the Authentication Data Request from the satellite, HSS in the GND generates Authentication Vector(s) as defined in clause 6.3.2 in TS 33.102 [7]. Authentication Vector consists of KASME, RAND, AUTN, and XRES.

6. The GND sends an authentication response back to the satellite that contains Authentication Vector(s).

PHASE 3. (Service link is available, Feeder link is unavailable)

The satellite in this PHASE may be different from the satellite in PHASE 1.

7. The UE retries the network connection by transmitting the Attach Request. This message can be protected using the similar method to step 2.

8. The satellite sends Authentication Request including AUTN and RAND.

9. At the receipt of the RAND and AUTN, the UE verifies the freshness of the received values by checking whether AUTN can be accepted as described in TS 33.102 [7]. If so, the UE computes a response RES.

10. UE responds with Authentication Response message including RES in case of successful AUTN verification. In this case, the UE computes KASME from CK, IK, and serving network's identity.

11. The satellite checks that the RES equals XRES. If so, the authentication is successful. As a result of the authentication and key agreement, an intermediate key KASME is shared between the UE and the satellite.

12. NAS SMC procedure is performed between the UE and the satellite.

6.25.3	Evaluation

This solution satisfies all of the potential security requirements in KI#1.

In particular, before the AKA procedure between the UE and the network is completed, the satellite and the UE check the authenticity of each other using the symmetric cryptosystem, which prevents the threats of DoS attack and false satellite attack.

To achieve this, the following changes are needed:

- eNB on a satellite generates and verifies MAC.

- UE generates and verifies MAC.

Therefore, this solution might require slightly more computational resource, but it will eliminate waste of storage resource of a satellite, which causes DoS attack captured in KI#1.