3GPP TSG-SA3 Meeting #117	     S3-243662

Maastricht, Netherlands, 19 – 23 August 2024

Source:	Google, Cisco Systems

Title:	Proposed solution for Secure Transport of Messages

Document for:	Approval

Agenda Item:	5.4

# 1	Decision/action requested

Approval of a new solution to address KI#2 in TR 33.776.

# 2	References

[1]	3GPP TR 33.776: “Study of ACME for Automated Certificate Management in SBA”

[2]	IETF RFC 8555: “Automatic Certificate Management Environment (ACME)”, 2019

[3]	3GPP TS 33.310: “Network Domain Security (NDS); Authentication Framework (AF)”

# 3	Rationale

This contribution proposes a solution that addresses Key Issue #2 - Secure transport of messages, as defined in TR 33.776 [1]. The solution is based on use of the ACME protocol, as defined in RFC 8555 [2].

# 4	Detailed proposal

***	BEGINNING OF CHANGES (all text new) ***

## 6.x	Solution #x: Using ACME protocol for secure transport of messages

### 6.x.1	Introduction

This contribution addresses key issue #2.

### 6.x.2	Solution details

The solution assumes that the 5G NF is issued with the operator CA’s root certificate, which is used to validate the ACME server’s TLS certificate.

This solution is based on RFC 8555 [2] wherein the communication between ACME client and the ACME server are done over HTTPS for authentication and confidentiality.

When an ACME client fetches a resource from an ACME server, the server authenticates the requester and verify any access control as described in RFC 8555 [2].

ACME for 5G SBA uses JWS based integrity protection as described in RFC 8555 [2].

ACME for 5G SBA uses nonces to protect messages against replay-attacks. An ACME server maintains a list of nonces that it has issued and requires any signed request from the client to carry such a nonce as described in RFC 8555 [2].

### 6.x.3 Evaluation

This solution addresses KI#2.

This solution impacts 5G core network function and 5G OAM system.

All exchanges initiated by the ACME client meet the requirement for confidentiality, integrity protection and replay protection. Once the client has established initial trust, messages can be considered mutually authenticated. Depending on the method of initial trust establishment, all messages could be considered mutually authenticated.

The server is always authenticated to the client prior to sending any data from the client, and no certificates are issued until mutual authentication is established.

The ACME server may need to initiate exchanges with the client, e.g. for http-01 challenge, or the DNS server, for example. It is FFS when these require protection, and how that would be achieved.

***	END OF CHANGES	***