3GPP TSG-SA3 Meeting #117	S3-243665

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Nokia, Nokia Shanghai Bell

# Title:	New KI on Security aspects related to enhancements of EAS and local UPF (re)selection

# Document for:	Approval

Agenda Item:	5.14

# 1	Decision/action requested

It is required to approve this contribution for TR 33.749

# 2	References

[1]	3GPP TR 23.700-49 “Study on Enhancement of support for Edge Computing in 5G Core network - Phase 3 (Release 19)

# 3	Rationale

The KI#2 of TR 23.700-49 [1] was intended to investigate whether and how to enhance EAS and local UPF (re)selection considering dynamic information related to EAS (load and N6 delay). SA2 has concluded that SMF local PSA UPF can be selected considering N6 delay, whereas the impact of N6 delay in EAS selection will be determined in normative phase. In any case, the N6 delay between 5GC N6 termination point (i.e., UPF) and the measurement endpoint at application side (i.e., EAS IP address(es)/designated IP ranges) is measured by leveraging existing mechanisms defined in IETF (e.g., ICMP, TWAMP, OWAMP, or other protocols).

A new KI is poprosed in TR 33.749 to analyze the potential security requirements associated with the use of those IP Performance Metrics (IPPM) defined protocols in IETF to measure N6 delays in the context of EAS and local UPF (re)selection.

# 4	Detailed proposal

******** START of CHANGES*********

## 5.3.X	Key Issue #Y: Security aspects related to enhancements of EAS and local UPF (re)selection.

### 5.3.X.1	Key issue details

Several enhancements in the procedures for EAS and local UPF (re)selection have been proposed in TR 23.700-49 [xx]. It has been concluded that N6 delay measurement is a factor of decision in those procedures. The N6 delay measurement between the termination points, i.e., UPF and EAS, is to be performed using existing IETF standard protocols (e.g., ICMP, TWAMP, OWAMP, etc.). The 5G network and the EDGE negotiate and exchange information about the termination points, protocols and parameters to be used, and then the SMF (“controller”) configures the UPF(s) to conduct the measurements as “senders” to the network devices/compute nodes/EAS(s) acting as “reflectors/receiver”. The UPF(s) will report the results to SMF.

The IETF RFCs corresponding to the IP Performanace Metrics (IPPM) defined protocols, such as RFC 4656, RFC 5357, RFC 8762, etc., include security considerations to be taken into account when those protocols are in use. RFC 2681 [yy] refers to two types of security concerns in these metrics: the potential harm caused by measurements, and the potential harm to the measurements.

The security risks associated with each protocol need to be tackled in the context of EAS and local UPF (re)selection, assuming that EDGE servers, such as the EAS, are placed outside the operator domain.

### 5.3.X.2	Security threats

Denial of Service: Potential attackers can exploit the capabilities of protocols like ICMP to overwhelm targeted servers like EAS (e.g., ping flood, smurf attacks, etc.).

Spoofing and Theft of service: Without authentication between, the termination points, the client may fake the source address of other device and use the measurement services produced by the server/reflector/receiver.

Tampering: The measurement traffic can be compromised by for example a man-in-the-middle entity, which can inject fake measurement traffic, or give to that a different priority, etc. Consequently, the selection of the server (UPF or EAS) could be biased by the attacker to their interest, leading in the worst-case scenario to the selection of a UPF or EAS already compromised and/or under their control.

### 5.3.X.3	Potential security requirements

Editor’s Note: The security mechanisms of the protocols to be used in N6 delay measurement are ffs.

******** END of CHANGES*********