3GPP TSG-SA3 Meeting #117	S3-243672

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Huawei, HiSilicon

# Title:	Sol#2 update to address ENs

# Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

Approve the pCR proposal for TR 33.759

# 2	References

[1]	3GPP TR 33.759

# 3	Rationale

This contribution proposes text to address ENs for Sol#2 in TR 33.759 [1].

# 4	Detailed proposal

pCR

***  BEGINNING OF CHANGES  ***

## 6.2	Solution # 2: UUAA supporting multiple USS

### 6.2.1	Introduction

This solution addresses the key issue #1.

The solution adapts the UUAA procedure in TS 33.256 [4] to support multiple USS. By and large, there are two changes: the UAV’s request and the UAS NF’s authentication requests/results are revised to support more than one USS.

### 6.2.2	Solution details

With reference to the clause 5.2.1.2/5.2.1.3 in TS 33.256 [4], the amended UUAA steps at registration/PDU session establishment respectively are shown below to support multiple USS:

1. The AMF/SMF triggers UUAA based on the CAA-Level UAV ID(s) provided by the UAV. The UAV may provide more than one USS addresses to indicate more than one USS is involved in the UUAA.

The UAV may have one unique CAA-Level UAV ID shared amongst multiple USS. In this case, one CAA-Level UAV ID is provided by the UAV. Alternatively, the UAV may also have one CAA-Level UAV ID assigned per USS. In this case, more than one CAA-Level UAV IDs are provided by the UAV.

NOTE 1: it is also allowed for UAV to provide single USS address in a UUAA procedure. This implies  one UUAA procedure per USS identified by the USS address.

2. The AMF/SMF sends a message to UAS-NF as in TS 33.256 [4].

3-4. The UAS NF resolves the USS address based on CAA-Level UAV ID or uses the provided USS address. If more than one USS addresses are resolved, the UAS NF sends an Authentication Request to each of the USS. The procedures of sending each of the Authentication Requests and the follow-up steps can follow the steps described in TS 33.256 [4].

5. The UAS NF can store multiple UUAA results (one for each USS) for each UAV.

6-8. The UAS NF sends USS-specific UUAA results (one for each USS) together with the USS Identifier to the UAV.

NOTE 2: It is possible to have different authorization results for different USS. The UAS NF treats UUAA results for different USS independently. For example, if UUAA is successful for USS-A but failed for USS-B, the UAV is only allowed to perform further UAS procedure with USS-A, but not allowed with USS-B.

NOTE 3: After successfully completing multiple UUAA, the UAS NF may instruct the UAV which USS to communicate first. Alternatively, the UAV determines the USS to communicate with based on the authorized route.

### 6.2.3	Evaluation

TBD

Editor’s Note: Alignment with SA2 conclusions for the support of multiple USS is FFS.

***  END OF CHANGES  ***