3GPP TSG-SA3 Meeting #117	S3-243674

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Huawei, HiSilicon

# Title:	Sol#2 evaluation

# Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

Approve the pCR proposal for TR 33.759

# 2	References

[1]	3GPP TR 33.759

# 3	Rationale

This contribution proposes text to evaluate Sol#2 in TR 33.759 [1].

# 4	Detailed proposal

pCR

***  BEGINNING OF CHANGES  ***

## 6.2	Solution # 2: UUAA supporting multiple USS

### 6.2.1	Introduction

This solution addresses the key issue #1.

The solution adapts the UUAA procedure in TS 33.256 [4] to support multiple USS. By and large, there are two changes: the UAV’s request and the UAS NF’s authentication requests/results are revised to support more than one USS.

### 6.2.2	Solution details

With reference to the clause 5.2.1.2/5.2.1.3 in TS 33.256 [4], the amended UUAA steps at registration/PDU session establishment respectively are shown below to support multiple USS:

1. The AMF/SMF triggers UUAA based on the CAA-Level UAV ID provided by the UAV. The UAV may provide more than one USS addresses to indicate more than one USS is involved in the UUAA.

NOTE: it is also allowed for UAV to provide single USS address in a UUAA procedure. This implies multiple rounds of UUAA procedures in support of multiple USS

2. The AMF/SMF sends a message to UAS-NF as in TS 33.256 [4].

3-4. The UAS NF resolves the USS address based on CAA-Level UAV ID or uses the provided USS address. If more than one USS addresses are resolved, the UAS NF sends an Authentication Request to each of the USS. The procedures of sending each of the Authentication Requests and the follow-up steps can follow the steps described in TS 33.256 [4].

5. The UAS NF can store multiple UUAA results (one for each USS) for each UAV.

6-8. The UAS NF sends USS-specific UUAA results (one for each USS) together with the USS Identifier to the UAV.

### 6.2.3	Evaluation

This solution addresses the key issue #1. It amends the UUAA procedures in TS 33.256 [4] in order to support multiple USSs. It has the following impacts:

- UAV: provides more than one USS addresses to the AMF/SMF. If the CAA-Level UAV ID is assigned per USS, the UAV provides multiple CAA-Level UAV IDs, one per USS.

- UAS NF: sends authentication requests to more than one USS and stores results for more than one USS.

Editor’s Note: How to handle conflicting parallel UUAA (e.g., CAA level UAV ID assignment, conflicting authorization result) is FFS.

Editor’s Note: How the UAV determines which USS to communicate with after successfully completing multiple UUAA is FFS.

Editor’s Note: Alignment with SA2 conclusions for the support of multiple USS is FFS.

***  END OF CHANGES  ***