3GPP TSG-SA3 Meeting #117	S3-243701

# Maastricht, Netherlands, 19 - 23 August 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.926 | CR | draftCR | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security Assurance Specification (SCAS) threats specific to SMSF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Indian Institute of Technology Bombay |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_SMSF |  |  |  |  |  | Date: |  |  | 23-08-2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New clauses identifying threats on the Diameter SGd and MAP SS7 interface of the SMSF |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clauses identifying threats related to the SGd and SS7 interface of the SMSF are added, along with a figure depicting interfaces between the SMSF and other network entities in the critical assets clause |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Legacy interfaces assets if not protected, could lead to security breach and network attacks |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Y.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of 1st Change ****************

# Y.2	Assets and threats specific to the SMSF

## Y.2.1	Critical assets

In addition to the critical assets of a GNP described in clause 5.2 of the TS 33.926 [7] document, the critical assets specific to the SMSF to be protected are:

-	SMSF Application;

-	NF and User Data: e.g. NF capabilities and events, network and user sensitive information (e.g., UeSMSContextData like supi, gpsi, ueLocation etc.), data retrieved from UDM, etc.

-	The interfaces of SMSF to be protected and which are within SECAM scope (Figure Y.2-1 depicts the different interfaces between SMSF and other network entities):

-	Service based interface for providing services to AMF, SMS-GMSC, IP-SM-GW, SMS-Router

-	Service based interface for consuming services from AMF, UDM

-    Reference point interface SGd (Diameter-based) with IP-SM-GW, SMS-GMSC, SMS- router

-    MAP-based SS7 interface with IP-SM-GW/GMSC/SMS- router

-	Console interface, for local access: local interface on SMSF

-	OAM interface, for remote access: interface between SMSF and OAM system

-	SMSF Software: binary code or executable code

NOTE 2:	SMSF files may be any file owned by a user (root user as well as non-root uses), including User account data and credentials, Log data, configuration data, OS files, SMSF application, NF and User data, or SMSF Software.

Figure Y.2-1: Interface connections between SMSF and other network entities

*************** End of 1st Change ****************

*************** Start of 2nd Change ****************

## Y.2.x	Threats related to Diameter-based SGd interface

### Y.2.x.y	Diameter filtering

- Threat name:  Diameter filtering

- Threat Category:  Denial of service.

- Threat Description:

If filtering is not applied at the Diameter SGd interface of SMSF, the SMSF may receive incoming messages that have been manipulated by an attacker. This could potentially lead to interface hijacking and, consequently, denial of service.

- Threatened Asset: SMSF application and Reference point interface SGd.

*************** End of 2nd Change ****************

*************** Start of 3rd Change ****************

## Y.2.x	Threats related to Diameter-based SGd interface

### Y.2.x.y	Diameter session information disclosure

- Threat name:  Diameter session information disclosure

- Threat Category:  Information disclosure, denial of service.

- Threat Description:

If the Diameter session is not an implicitly terminated session, it can possibly lead to session hijacking and consequently denial of service. Furthermore, if Diameter session IDs are not unique, then it can lead to unintended information disclosure due to incorrect session associations.

- Threatened Asset: SMSF and user data.

*************** End of 3rd Change ****************

*************** Start of 4th Change ****************

## Y.2.x	Threats related to Map-based SS7 interface

### Y.2.x.y	Map/SS7 interface protection

- Threat name:  Map interface protection

- Threat Category: Tampering, Information Disclosure.

- Threat Description: SMS related traffic is transported between the SMSF and the IP-SM-GW/SMS-GMSC/SMS-Router on Map-based SS7 interface. In roaming scenario, transportation is via Signaling Transfer Point (STP) node on Map interface. If the SMS data transported over the interface is not confidentiality protected, it can be subject to eavesdropping, and information leakage to unauthorized or spoofed parties. If the SMS traffic is not integrity protected, the SMS traffic can be tampered with.

If the protection of the Map-based SS7 interface is done using wrong security profiles, i.e., weak security algorithms or protocol versions with known vulnerabilities, then overall security of the data transported over SS7 is compromised.

- Threatened Asset: Network function and data.

*************** End of 4th Change ****************

*************** Start of 5th Change ****************

## Y.2.x	Threats related to Map-based SS7 interface

### Y.2.x.y	Map filtering

- Threat name:  Map filtering

- Threat Category:  Denial of service.

- Threat Description:

If filtering is not applied at the Map-based SS7 interface of SMSF, the SMSF may receive incoming messages that have been manipulated by an attacker. This could potentially lead to interface hijacking and, consequently, denial of service.

- Threatened Asset: SMSF application and Reference point interface SS7.

*************** End of 5th Change ****************