3GPP TSG-SA3 Meeting #117	S3-243365

Maastricht, Netherlands 19 - 23 August 2024

# Source:	OTD_US, MITRE-FFRDC, US National Security Agency

# Title:	KI#1, Solution #3 Updates: Data Collection

# Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

This pCR proposes to resolve Editor’s Notes and clarify procedures in Solution #1 of TR 33.794 [X].

# 2	References

[X]	3GPP TR 33.794 Study on enablers for Zero Trust Security

# 3	Rationale

This contribution resolves multiple Editor’s Notes in Solution #3 as well as expands the solution to meet KI#1 requirements.

# 4	Detailed proposal

SA3 is kindly requested to approve the below changes to TR 33.794 [X]

------START OF CHANGES------

------START OF FIRST CHANGE------

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

AMF	Access and Mobility Management Function

NF	Network Function

NRF	Network Repository Function

NRF-Sec	Network Repository Function - Security

OSF	Operator Security Function

SADF	Security Administration Function

SCP	Service Communication Proxy

SDCF	Security Data Collection Function

SDRF	Security Data Repository Function

SDPI	Security Data Point of Ingest

SMF	Session Management Function

------END OF FIRST CHANGE------

------START OF SECOND CHANGE------

## 7.3	Solution #3: New Data Collection NFs

### 7.3.1	Introduction

This solution addresses KI#1 Data exposure for security evaluation and monitoring. Specifically, it addresses the requirement: (2) Architecture to be used for exposure of data collected for security evaluation and monitoring of the 5G SBA.

The basic principle of this solution is to define security architecture elements that function as the security data points of ingest (SDPIs) which can be used by the operator’s security function (OSF) to execute policy decision points (PDPs). SDPIs, in this proposal can read all SBI related data on the NF and are configured to send this data (on a per NF basis per operator policy) to intermediate logical functions described below which then delivers the data to the operator’s security function (OSF). While not explicitly defined in this solution, it is assumed that the OSF may also collect data from the OAM system as described in TS 23.288 [13] clause 6.2.3 and in SA5 specifications.

SDPI authorization is achieved through the NRF-Sec, a sub-function of the NRF which is responsible for storage of NF SDPI functionality and authorization of the SDPI via reuse of the Nnrf_NFManagement interface. SDPI communications security is achieved through mandatory use of TLS from SDPI to next-hop function (e.g. SDPI to SDCF).

The following functions are defined:

1.	Security Data Collection Function (SDCF) which is responsible for consuming the data collected from either SDPIs or via other existing interfaces.

2.	Security Data Repository Function (SDRF) which is responsible for storage of the SDCF data and is configured by the operator.

3.	Security administration function (SADF) which is configured by the operator and acts as an intermediary, coordinating, authorizing, and monitoring the tasks present at the various SDPIs present in NFs.

4.	NRF-Sec which is responsible for the on boarding and storage of the SDPI profile for the NF in the NRF.

NOTE 1:	The conclusion phase could determine if some of the functions defined in the bulleted list can be combined.

The OSF remains outside the scope of 3GPP but is expected to provide logic of PDP. The SDRF is defined to act as the producer which then interfaces via an internal mechanism to the OSF.

There could be one or many SDRFs. The solution is based on the existing SBA allowing SDCF and SDRF to communicate via direct communication or use of the NRF and or SCP as defined in 3GPP 33.501 [4].

NOTE 2:	If an NRF and/or SCP is used, these might be separate physical entities on the SBA network to reduce the impact of security network being compromised. This is an operational deployment decision and is outside the scope of 3GPP.

NOTE 3: This solution assumes reuse of existing interfaces and message types (e.g. SADF to SDCF using Nnwdaf and SDCF to SDPI using nNF). Specific message types will be defined in normative phase.

NOTE 4: This solution assumes that when data collection is specifically performed for security purposes, it assumes the same level of sensitivity as native security information. This solution also assumes that the security layer is both isolated and separated from the network operation layer.

### 7.3.2	Solution details

#### 7.3.2.1	General

The following clauses illustrate SDPI registration and data collection rule configuration, data collection, and data delivery.

Clauses 7.3.2.2, 7.3.2.3 and 7.2.3.4 depict a Security Layer to delineate between normal 5GC signalling (i.e., Network Layer) and signalling for security purposes (i.e., Security Layer). Security specific functions, see clause 7.3.1 of the present document, reside in the security layer are not exposed to the generalized network layer. SBI communication in the Security Layer may reuse existing SBI security procedures defined in TS 33.501 [4] clause 13.

#### 7.3.2.2	SDPI registration and data collection rule configuration

Figure 7.3.2.2-1: SDPI Registration and Data Collection Rule Configuration

- When an NF registers or updates its registration in the NRF/NRF-Sec it includes details of its SDPI functionality.

- NRF/NRF-Sec updates NF profile with SDPI functionality indicator.

- SDPI registers to NRF-Sec.

- The operator provisions OSF policy including data collection rule sets and sends to the SADF.

NOTE: Step 3 may happen at any time, potentially before the NF registers to the NRF.

- The SADF performs NF/SDPI discovery via the NRF/NRF-Sec, establishing secure communications channel with the SDCF.

- The SADF forwards the data collection policy rule set to the SDCF

- The SDCF sends data collection rule sets to the NFs/SDPIs from which data may be collected.

- Configuration of NF data collection (see clause 7.3.2.2) begins with data delivery configuration steps defined in clause 7.3.2.3 of the present document.

#### 7.3.2.3	Data Collection

Figure 7.3.2.3-1: Data collection request

0. The operator provisions OSF policy including SDPI rule sets.

1. The OSF sends a data collection request  message to the SADF identifying the SDPIs (e.g. SDPI in Nf1, NF2, NF3) it requires to be activated, storage criteria, and event generation reporting schemas.

NOTE 1:	This interface between the SADF and the OSF is not a 5G SBA interface and is outside the scope of 3GPP. However, a secure channel (e.g. TLS) is setup between them.

2.	SADF sends a data collection request message to the SDCF. The message shall include the NF types and NF IDs that the OSF wanted analytics information from. E.g. NF1, NF2 and NF3.

2a.	SDCF receives and processes the data collection request message sent from the SADF.

NOTE 2: The ability for the SADF to send specific NF Instance IDs and/or NF types to the SDCF is dependent upon configuration of NF exposure via the NRF. This solution assumes that the SADF is responsible for the configuration of the SDPI instances via nNF based upon data collection rule configurations received from the SADF.

3.	SDCF sends a data collection request message to all relevant SDPIs in the identified NF types indicated by the SADF, derived from the SDPI rules set by the OSF.

This solution assumes a native capability for the SADF to translate instructions/rule sets provided via the OSF (via nOSF interface) and generate nNWDAF SBI messages (e.g. NnwdafAnalyticsInfo Request/Response).

NOTE 3:	Operator policy and or configuration identifies the type of interface to use.

#### 7.3.2.4	Data delivery

Figure 7.3.2.4-1: Data delivery

1a/b.	SDCF receives data payload from an NF SDPI (e.g. NF1, NF2). All data payloads include a UUID to identify the unique payload, SDPI ID, NF ID and NF type from which it originated.

NOTE 1: This solution assumes the reuse of data payload structures of TS 23.288 [13] clause 6.2. Further data payload structure definitions may be defined by SA2 or SA5.

NOTE 2: This solution assumes reuse of existing interfaces (e.g. SDPI to SDCF using nNF).

2.	SDCF decides, based on configuration of data payload delivery endpoint sent in the data collection request message, to send data payload it has received to SDRF for storage and future retrieval by the SADF or directly by OSF. The OSF data collection policy delivery rules are received at the SDCF from the SADF as in Step 2 of clause 7.3.2.2. If delivery rule is set for SDRF delivery, the SDCF sends the data payload it has received from (e.g. NF1, NF2) to SDRF.

3.		SDRF performs necessary validation to ensure that the NF sending the data payload (SDCF) is allowed to send SDRF data payload and that SDCF is allowed to send data payload from NF instance Id from step 1a) and from NF instance ID from step 1b).

NOTE 3:	SDCF could only send data payload from one NF e.g. AMF and SDCF send the other NFs analytic payload e.g. SMF data later.

Data payload deduplication when data is sent over more than one interface is left to OSF implementation.

4.	SDRF decides, based on data collection policy delivery rules received by the SADF from the OSF and configured in the SDRF or by explicit instruction from to OSF, to send data payload it has received to the OSF.

5.	SDRF sends the identified data payload if received from SDCF to the OSF.

NOTE 4:	OSF may configure data delivery based upon time, data volume, or specific trigger (e.g. event type) configured based on operator policy.

### 7.3.3	Evaluation

The capabilities of the OSF to interpret, use, and configure actions based upon received security data are out of 3GPP scope. This solution provides a basic framework to send data collection policies, configure data payload collection, and perform data delivery to the OSF.

This solution has no direct external impact on other working groups (e.g. SA2) as the solution is self-contained and can be executed at the security layer. Reuse of existing SBI messages/interface definitions (e.g. Nnwdaf) between the new data collection NFs limits the need to define new message types and minimizes overall complexity.

This solution does not seek to modify the NWDAF, limiting impact on existing specifications (e.g. TS 23.288 [13]), nor use the NWDAF for security purposes.

------END OF SECOND CHANGE------

------END OF ALL CHANGES------