SA WG3 Meeting S3#117	S3-242872

Maastricht, Netherlands 19 - 23 August 2024                    revision of S3-24wxyz

Source:	Nokia, Nokia Shanghai Bell

Title:	Update Sol3 on Support of Third Party specific User Identities in IMS

Document for:	Approval

Agenda Item:	5.2

Work Item / Release:	FS_NG_RTC_Ph2/Rel19

# 1	Decision/action requested

Approve the solution update added to TR 33.790

# 2	References

[1]	3GPP TR 23.700-77 Study on system architecture for next generation real time communication services Phase 2

[2]	3GPP TR 33.790 Study on the security support for the Next Generation Real Time Communication services phase 2

# 3	Rationale

The contribution proposes an update to existing solution#3 on third party identity authentication and authorization to address Editor notes and add evaluation. The contribution also updates the workflow to align with SA2 conclusion.

# 4	Detailed proposals

Updated content in the change part is new.

# * * * * First change* * * *

### 6.3.2	Solution detail

There are several options how and where Third Party specific user identities are signed and verified, which allow for different deployment scenarios, e.g. using UNI or NNI interface between Third Party and IMS network, with different levels of impact to the Third Party network and the IMS network and with different levels of trust relationship between both.

Generally, the HSS stores one or several URL(s) pointing to resources on Web servers where Third Party specific user identities and data are stored. This includes URL(s) pointing to Rich Call Data (RCD URL) as described above or pointing to any other user or Third Party specific data. Alternatively, HSS may store RCD server address. Storing just URL(s) or RCD server address in the HSS avoids potential misusing a Third Party specific user identities that no longer belongs to an UE to initiate IMS calls (e.g., the user uses the identities allocated by a particular company even after leaving it), and possibly frequent updates to the data based on request from the Third Party network and avoids defining Third Party specific data formats in HSS. Nevertheless, the HSS may also store additional data in the subscription of a Third Party subscriber like caller name, organization information, job title, and location information. The URL(s) and possibly other data are fetched from the HSS by the CSCF or IMS AS depending which entity invokes the signing. Based on operator policies, the source IMS AS may use the RCD URL received from the HSS to fetch Rich Call Data from a server that can be in the operator domain or external in the Third Party network and provide these data or the RCD URL in SIP signalling (SIP INVITE) towards the terminating party. The Rich Call Data information or URL is used by the Signing AS for signing the RCD PASSporT and by the Verification AS to verify the signed RCD PASSporT.

Based on operator policies, the terminate IMS AS will invoke verification of the RCD information or RCD URL included in the incoming SIP INVITE request with a verification AS, and forward the RCD information or RCD URL to the terminating UE.

The terminating UE may retrieve third party user identity information from the third party server based on the received RCD URL or RCD information.

The SIP header extensions (e.g. Call-Info header) required to transfer Third Party specific user identity information are defined by stage 3.

The procedures to sign and verify PASSporT tokens follow the descriptions in TS 24.229 [4] with the main difference that besides telephone numbers also other information as described in draft-ietf-sipcore-callinfo-rcd-09 [8] and draft-ietf-stir-passport-rcd-26 [6] can be used for signing and verification.

Figure 6.3.2-1: Third Party Identity signing and verification workflow

1.	The originating UE sends a SIP INVITE that contains the IMPU of the calling UE and optional Third Party specific user identity (or third party identity).

2.	The CSCF forwards the SIP request to the IMS AS.

3.	The IMS AS checks with HSS if the calling user (IMPI or IMPU based) is authorized to use the third party identity based on subscription. The association between IMPU/IMPI and third party ID/RCD URL is pre-configured in HSS as subscription data.

4a.	The IMS AS/CSCF retrieves Rich Call Data (RCD) server address, or RCD URL, or RCD information of the third party identity from HSS. HSS may return RCD server address, RCD URL pointing to the RCD on Web servers or concrete RCD, like caller name, job title, organization, and location information, etc., based on deployment option. The CA root certificate used to sign the certificate(s) of https of the RCD URL/URIs may be included in the response.

NOTE:	If HSS returns concrete RCD to the IMS AS in this step, HSS should fetch the RCD from the third party database in advance based on RCD URL/third party identity associated to the IMPU/IMPI.

4b. Optionally, the IMS AS/CSCF may retrieve RCD information of the third party identity from third party database based on RCD server address and the third party identity, or RCD URL.

5.	The IMS AS/CSCF calls STI-AS to sign the SIP header, e.g. call-info, which including RCD URL or RCD information of the third party identity.

6.	The STI-AS returns the signed SIP header back to the IMS AS/CSCF.

7.	The IMS-AS/CSCF forward the SIP INVITE to the terminating IMS subsystem through CSCF which including signed RCD URL or RCD information of the third party identity, and optional the CA root certificate received in step 4.

8.	The terminating IMS subsystem invokes the STI-VS to verify the signed RCD URL or RCD information.

9.	If verification is successful,

the terminating IMS subsystem sends SIP INVITE to terminating UE which including the RCD URL or RCD information if verification is successful in step 8, the CA root certificate used to sign the certificate(s) of https of the RCD URL/URIs may be included in the message. Otherwise, terminating IMS subsystem may send SIP INVITE to terminating UE without including RCD.

10. Optionally, the UE-B fetches RCD information from third party databased based on the RCD URLs/URIs received in step 9 after verifying the https of the URLs/URIs with the root certificate received in step 9.

11.	The terminating UE sends 18X/200 to originating IMS subsystem and to the originating UE.

# * * * * Second change* * * *

### 6.3.3	Evaluation

The solution addresses requirements of Key issue #1 which leverages STIR/SHAKEN framework, RCD and PASSporT defined in IETF to verify and authorize the third-party specific user identities, and support integrity protection of the third-party specific user identities on the originating side and terminating side. The solution also supports the terminating UE to securely retrieve RCD information from the third party database.

The solution is aligned with SA2 conclusion on KI#4 of TR 23.700-77.

# * * * * End of changes * * * *