3GPP TSG-SA3 Meeting #117	S3-242881

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Huawei, HiSilicon

# Title:	Sol#3 update to address ENs

# Document for:	Approval

Agenda Item:	5.11

# 1	Decision/action requested

Approve the pCR proposal for TR 33.759

# 2	References

[1]	3GPP TR 33.759

# 3	Rationale

This contribution proposes text to address ENs for Sol#3 in TR 33.759 [1].

# 4	Detailed proposal

pCR

***  BEGINNING OF CHANGES  ***

## 6.3	Solution # 3: Pairing authorization supporting multiple USS

### 6.3.1	Introduction

This solution addresses the key issue #1. The solution adapts the pairing authorization procedure in TS 33.256 [4] to support multiple USS.

Pairing authorization can occur during the UUAA-SM procedure or after a successful UUAA. For the former, it is assumed that UUAA has been amended to support multiple USS and no further change is needed. This solution is to address the latter scenario.

### 6.3.2	Solution details

With reference to the clause 5.4.2 in TS 33.256 [4], the amended UUAA procedure at PDU session establishment/modification is shown below to support multiple USS:

1.The UAV includes the USS address(es) of the UAV-C to pair in the PDU session establishment/modification request, in addition to other information as specified in TS 33.256 [4].

NOTE 1: Although the pairing information may contain USS information, it is transparent to the UAS NF. The UAS NF may send to a different USS for pairing authorization.

2. The SMF invokes the pairing authorization procedure and the UAS NF exchanges authorization messages with and receives results with the USS identifier from the USS identified by the USS address(es) in step 1. The rest are the same as in the step 2 in the clause 5.4.2 of TS 33.256 [4].

3. The SMF informs the UE the paring authorization result which may include USS identifier in addition to information specified in the step 3 in the clause 5.4.2 of TS 33.256 [4].

NOTE 2: Different USS may have different pairing authorization results. The UAS NF treats pairing authorization results for different USS independently, i.e., USS-specific paring.

### 6.3.3	Evaluation

Editor’s Note: Alignment with SA2 conclusions for the support of multiple USS is FFS.

***  END OF CHANGES  ***