3GPP TSG-SA3 Meeting #117	S3-242887

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Huawei, HiSilicon

# Title:	Sol#19 evaluation

# Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

Approve the pCR proposal for TR 33.700-32

# 2	References

[1]	3GPP TR 33.700-32

# 3	Rationale

This contribution proposes evaluation text for Sol#19 in TR 33.700-32 [1].

# 4	Detailed proposal

pCR

***  BEGINNING OF CHANGES  ***

## 6.19	Solution #19: User privacy protection

### 6.19.1	Introduction

This solution addresses the key issue #2.

The solution protects a user identifier (UID) used in 3GPP system by replacing it with the concealed version, i.e. user identifier concealed (UIC). Since UID is not in the realm of 3GPP systems, the mapping between the UID and UIC is out of scope of 3GPP system, i.e. UIC and how UID is mapped to UIC is transparent to the 3GPP systems.

In addition, the mapping between UIC and UID is not fixed and changed from time to time to avoid the user being tracked or linked.

Editor’s note: ffs to clarify how and who to conceal UID.

Editor’s note: In case of PDU session request, how we will ensure the privacy of the user ID considering the mobility scenario.

### 6.19.2	Solution details

A user authentication procedure is exemplified below to illustrate how to use UIC, instead of UID to protect user privacy. It is notable that the authentication procedure is made generic as much as possible to accommodate different authentication methods (this solution is not meant to address authentication procedure).

The exemplified user identity authentication and authorization (UIAA) procedure with user privacy protection is as follows:

1.The UE sends a Registration Request to the AMF.

2. The AMF triggers the UIAA procedure.

3: The AMF sends the AAA-Server the UIAA message with GPSI included.

Editor's note:	With the UIC transparent to the AMF, how the AMF determines whether, for whom, and how to trigger user authentication is FFS.

4. The AAA-S and the UE perform user authentication and authorization with respect to the user, e.g. the user is identified as UIC between AAA-S and the UE. However, the UIC is transparent to the AMF. There can be multiple message exchanges between UE and AAA-S, depending on authentication method used. The authentication method is out of scope of 3GPP.

5. The AAA-S sends the authentication results to AMF, which stores the result identifying the user by UIC, the concealed version of UID. The UIC should be changed for every authentication to avoid being tracked or linked to previous authentication.

6. The AMF sends registration accept to the UE.

Figure 6.19.2-1 illustration of user privacy protection in a user authentication procedure

### 6.19.3	Evaluation

This solution addresses the key issue #2.

This solution provides user privacy protection by using a concealed user identifer (UIC) in place of the real user identifier (UID). The privacy of UID is a end-to-end protected between the UE and the AAA-Server. The 3GPP system uses the protected version, i.e., the UIC in the 3GPP procedures. Therefore, the 3GPP system does not need to provide an additional protection mechanism. The UIC may be updated to the 3GPP system from time to time to avoid the user being tracked or linked.

***  END OF CHANGES  ***