3GPP TSG-SA3 Meeting #117	S3-242904

Maastricht, Netherlands  19 - 23 August 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.535 | CR | 0216 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to update AKMA service disable procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AKMA |  |  |  |  |  | Date: |  |  | 2024-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is a AKMA service disable failure problem described in S3-241971 in May meeting. In following case, the AAnF may fail to disable the AKMA service when the local policy is not allowed. - UE performs registartion to PLMN#1 and primary authentication. AUSF sends the generated A-KID#1 and KAKMA#1 to the AAnF - UE initiates communication with the AF, which includes the derived A-KID#1 in the Application Session Establishment Request - AAnF sends A-KID#1 and KAF to AF. Afterwards, the AF sends the Application Session Establishment Response to the UE - UE is getting registered in PLMN#2 and perform another round of primary authentication. AUSF sends the generated A-KID#2 and KAKMA#2 to the AAnF - If AAnF detects and restricts the AKMA access in the PLMN#2, the AAnF shall send notifications to the subscribed AF(s), which includes A-KID#2. However, AF is still using A-KID#1 for AKMA service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The AAnF includes SUPI together with A-KID in the AKMA service disable notification message to the AF. The AF can use the SUPI to do the service disable if the A-KID is not valid for AF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Failure of AKMA service disabling |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8, 7.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************* Start of 1st Changes *************

6.8	Notification about AKMA service disabling

This procedure is used when the AKMA sessions have already been started (before roaming was detected), and as soon as PLMN change is detected at the AAnF, the AAnF may execute this procedure based on the roaming policy.

Figure 6.8.1-1: AAnF notification to AF about AKMA service disable

1.	 UE registers with a (H)PLMN.

2.	 UE is accessing the AF and key material is provided to AF as described in 6.2.1. While accessing the AAnF, AF may also provide the Notification URI.

3. UE is getting registered in a VPLMN and AAnF detects the PLMN change via the Nudm_EventExposure_Notification received from UDM.

4. AAnF determines if AF(s) have subscribed to receive notifications for AKMA service disabling and roaming policy is configured and restrict the AKMA access in the VPLMN; if yes, steps 5 and 6 are executed. Otherwise, steps 5 and 6 are skipped.

5. If AF(s) are determined at step 4, the AAnF shall send notifications to the subscribed AF(s) about AKMA roaming via Naanf_AKMA_ServiceDisableNotification with A-KID and SUPI/GPSI.

6. The AF shall send the response and based on the notification and internal policy, the AF may stop the UE service, may stop the encryption.

NOTE: By stopping the encryption (e.g., TLS 1.2 NULL cypher negotiation), LI interception could work in the VPLMN.

************* End of 1st Changes *************

************* Start of 2nd Changes *************

7.1.6	Naanf_AKMA_ServiceDisableNotification service operation

Service operation name: Naanf_AKMA_ServiceDisableNotification

Description: AAnF notifies the NF consumer about AKMA service disable

NOTE:	The AF is implicitly subscribed to receive Naanf_AKMA_ServiceDisableNotification service operation.

Input, Required: A-KID, SUPI/GPSI

Input, Optional: None

Output, Required: None

Output, Optional: None

************* End of 2nd Changes *************