3GPP TSG-SA3 Meeting #117	S3-242930

Maastricht, the Netherlands, 19th - 23th August 2024

# Source:	ZTE Corporation

# Title:	Evaluation for solution #1

# Document for:	Approval

Agenda Item:	5.18

1	Decision/action requested

This contribution proposes evaluation for solution #1.

2	References

[1]	3GPP TR 23.700-21: " Study on Application enablement architecture for mobile metaverse services "

3	Rationale

This contribution proposes evaluation for solution #1 of TR 33.721.

Solution #1 proposes to reuse the SEAL identity management service to perform UE authentication and issue access token, and use SEAL LM server to verify the token and provide the requested spatial localization service.

Currently, SA6 has concluded on their KI#1: Enabler support for managing spatial anchors in TR 23.700-21. It is concluded to define new SEAL server to support spatial anchor management:

For Key issue #1 Enabler support for managing spatial anchors, it is concluded to define new SEAL server to support spatial anchor management. The new SEAL server will consider to re-use the architecture as specified in option#1.

for Key issue #1 (Enabler support for managing spatial anchors): Solution #1, Solution#2 and Solution #3 can be consider for normative work with possible merge of the procedures as specified in clause 10.2.1. All solutions will be adapted to the conclusion of the architecture option in the normative work.

According to their conclusion, spatial anchor management service will be provided by a new SEAL server rather than using the existing SEAL LM server. Therefore, this contribution proposed to update solution #1 based on SA6’s conclusion and add text for the evaluation to clarify that the proposed solution works for SEAL service authorization regardless of the specific SEAL server.

4	Detailed proposal

**** Start of Changes****

6.1	Solution #1: Support for spatial localization service authorization

6.1.1	Introduction

This solution is for KI #1 and addresses the security requirements for authorizing UE to access spatial localization services. This solution is based on the  SEAL identity management (SIM) service to perform UE authentication and authorization.

    - Solution details

Before getting authorization to specific service, the VAL UE authentication is executed by the SIM-S as described in TS 33.434 [4]. After successful authentication, the SIM-C requests and receives an access token from SIM-S as shown in Figure 6.1.2-1.

Figure 6.1.2-1: Get Access Token

- User Authentication is completed between VAL UE and the SIM-S.

- The VAL UE sends an access token request to the SIM-S, including the identity of the VAL UE and the specific spatial localization service the UE requests to access.

- The SIM-S authorizes the VAL UE for the requested service and provides access token for the VAL UE.

With the received access token, the VAL UE can request for spatial localization service from corresponding SEAL server supporting the requested service. The procedure of getting spatial map for metaverse application is shown in Figure 6.1.2-2.

Figure 6.1.2-2: Get Spatial Map

- A secure channel is established between SEAL client and SEAL server. Subsequent communication makes use of this channel.

1. The VAL UE sends a request message containing the access token to the SEAL server to get the spatial map via SEAL LM client.

2. On receiving the service authorization message, the SEAL server validates the access token.

3. If the access token is valid, the SEAL server provides the spatial map information to the VAL UE via SEAL client. Otherwise, the response included the failure cause indicating that the token is invalid.

The same procedure can also be applied for getting spatial anchor and any other spatial localization services provided by other SEAL servers by changing the request service.

Editor’s Note: The application enablement architecture for metaverse services is to be aligned with SA6.

6.1.3	Evaluation

This solution satisfies the potential security requirements of authorizing UE for access spatial localization services.

In particular, the SIM-S performs UE authorization by issuing UE access token for the requested spatial localization service. The access token is included in the service request sent by UE and verified by the corresponding SEAL server. If the verification is successful, the UE will be granted use of the requested spatial localization service.

NOTE: The SEAL server(s) supporting spatial localization services are to be defined in SA6.

To achieve this, the following changes are needed:

- Spatial localization related SEAL service IDs are assigned and provisioned to the SIM-S.

Editor's Note: Further evaluation is FFS

**** End of Changes****