3GPP TSG-SA3 Meeting #117	S3-242995

Maastricht, NL, 19 - 23 August 2024

# Source:	Apple

# Title:	Discussion Paper on Security Key Handling for Inter-CU LTM in non-DC cases

# Document for:	Discussion

Agenda Item:	4.12

# 1	Decision/action requested

It is requested that to endorse the Proposals

# 2	References

[1]	 R2-2404037: " LS on security handling for inter-CU LTM in non-DC cases "

# 3	Rationale

3.1 R19 Inter-CU LTM

RAN2 sent LS S3-241773 on security handling for inter-CU LTM in non-DC cases, asking SA3 to consider the needed signalling between participating network nodes for each option and inform RAN2 if any of the above options is not feasible or not acceptable from security perspective and provide modifications that could make those options feasible and acceptable. Besides, RAN2 also requests SA3 whether, for each option, the change of security algorithm or the change of key set indicator is to be supported for inter-CU LTM.

Based on the basic LTM procedure and the options provided in RAN LS, there are multiple security issues to be addressed in SA3:

3.2 Security issues to be addressed:

- UE needs configuration of NCC(s). Options are as following:

  - Option 1.1: UE gets configuration (one NCC or a list of NCCs) from unprotected MAC CE

  - Option 1.2: UE gets configuration (one NCC or a list of NCCs) in protected RRC messages from source gNB.

MAC CE has no security protection, which will lead to the eavesdropping and tampering of the configurations. Even though it sacrifices the security, leveraging MAC CE has the benefit of simplifying the NCC delivery. The tampering of the NCC will lead to the wrong configuration, i.e. the key mismatch, the LTM needs to be re-configured again. The result is the whole efficiency is decreased. From this perspective, security protected RRC message is preferred.

However, LTM Cell Switch Command MAC CE is already defined in TS 38.321 clause 6.1.3.75, which includes multiple fields: Target Configuration ID, Timing Advance Command, TCI State ID, UL TCI state ID, SS/PBCH index, PRACH Mask Index, Repetition number, etc. The modification of these fields will also cause the LTM handover failure. In other words, adding NCC in this MAC CE (or a define a new MAC CE) will not make the security level of LTM Cell Switch Command MAC CE lower.

Proposal1: Using MAC CE to deliver the NCC is not recommended from security perspective. However, using MAC CE to deliver NCC is not making the LTM Cell Switch Command MAC CE less secure, since this MAC CE is already used to deliver other configuration information. Therefore, it is proposed to leave to RAN2 to make the final decision.

- When UE is switching between 2 same cells, could UE use horizontal key derivation?

When UE is switching between CU A and CU B, step 1: CU A to CU B, step 2: CU B to CU A.

Following the HO procedure, in step A, CU A will deliver the key to CU B, in step 2, CU B will deliver the key to CU A for the HO. The key derivations are based on different NCC and K_gNB. The horizontal key derivation is used when the gNB has no new fresh {NH, NCC} pair, which is not applicable for this case. Thus horizontal key derivation is not a suitable mechanism here if NCC is updated after every switch.

Proposal 2: Horizontal key derivation is not a suitable mechanism here when the NCC is updated after every switch.

- How to avoid synchronisation failure?

The synchronization failure avoidance should consider the detailed security procedures. If the solution is involving the list of NCC and the AMF, then UE, gNBs and AMF should follow the preconfigured sequence to use the NCC for the next switch. One possibility is to reuse the NGAP PATH SWITCH COMPLETE to indicate the next NCC used in last switch.

If the solution is not involving the AMF, then MAC CE or RRCReconfiguration can be used to indicate to UE about which NCC to be used for next switch. This NCC is provided from AMF to gNB through legacy NGAP PATH SWITCH ACK message.

Proposal 3: Depending on the detailed security procedure, the synchronisation failure can be avoided using different approaches, MAC CE, RRCmessage (i.e.RRCReconfiguration) or NGAP PATH SWITCH/ACK.

- Is it necessary to have AMF impact?

If a list of NCC is going to be used, then AMF must be impacted, since the list of NCC needs to be provisioned from AMF to gNB and UE. However, it is still feasible if AMF still uses one {NH, NCC} pair after UE switches to a CU. The source CU can use this NCC it received from PATH SWITCH ACK to derive 7 keys for other candidate CUs. Vertical key derivation is used here following TS 33.501 key derivation for handover procedure while 2-hop security can be achieved.

Proposal4: Without violating security principles, a mechanism without AMF impact is feasible.

- Should the list of NCCs to be used or one single NCC is sufficient.

Both options are feasible. Using one single NCC and following the legacy NCC configuration procedure (i.e. NGAP PATH SWITCH COMPLETE/ACK) could satisfy the basic requirement for LTM. Configuring a list of NCC will increase the complexity and impact of AMF, gNB and UE.

Proposal5: Both options are feasible. One single NCC is the simplest approach to realize the LTM security key derivation.

Based on the discussion above, there are 2 options for the LTM security procedure.

Option#1: MAC CE based NCC delivery.

The exemplary of the option#1 (details in S3-242997)

Option#2: RRC based NCC delivery.

The exemplary of the option#2 (details in S3-242998):

# 4	Detailed proposals

Proposal 0: It is proposed SA3 should work on those questions above one by one to get consensus from security perspective.

Proposal1: Using MAC CE to deliver the NCC is not recommended from security perspective. However, using MAC CE to deliver NCC is not making the LTM Cell Switch Command MAC CE less secure, since this MAC CE is already delivering other configuration information. It is proposed to leave to RAN2 to make the final decision.

Proposal 2: Horizontal key derivation is not a suitable mechanism here when the NCC is updated after every switch.

Proposal 3: Depending on the detailed security procedure, the synchronisation failure can be avoided using different approaches, MAC CE, RRCReconfiguration or NGAP PATH SWITCH/ACK.

Proposal4: Without violating security principles, a mechanism without AMF impact is feasible.

Proposal5: Both options (using one NCC or a list of NCCs) are feasible. One single NCC is the simplest approach to realize the LTM security key derivation.