3GPP TSG-SA3 Meeting #117 	S3-242998

# Maastricht, NL, 19 - 23 August 2024

# Source:	Apple

# Title:	RRC message based solution for LTM security

# Document for:	Approval

Agenda Item:	4.12

# 1	Decision/action requested

Approve the pCR to the living CR of LTM security

# References

# 3	Rationale

This contribution proposes a new LTM security solution, which is based on RRC message to deliver NCC.

# 4	Detailed proposal

*** Start of 1st Change ***

## 3.X	Solution #X: RRC message based solution to deliver the NCC(s)

### 3.X.1	Introduction

This solution proposes the procedure for LTM key derivation. The legacy handover procedure in TS 33.501 clause 6.9.2 will be reused as much as possible. RAN2 procedures on LTM (no DC) are used as the basis.

### 3.X.2	Details

Preassumption:

- In this solution, gNB1, gNB2 and gNB3 are the candidate cells, UE is moving from gNB1 to gNB2, then from gNB2 to gNB3.

- Source gNB1 and UE will use NCC1 for the next switch.

Figure 3.x.1-1. RRC message based LTM procedure

Step 0. Source gNB1 configures the UE, NCC1 is the next NCC to be used. gNB1 configures gNB2 and gNB3 as the candicate CUs.

Step 1. gNB1 sends the MAC CE to the UE to trigger the switch.

Step2. gNB1 derives K_gNB21 based on NCC1 and sends it to gNB2, the key derivation method follows TS 33.501 Annex A.11.

Step3. gNB1 derives K_gNB31 based on NCC1 and sends it to gNB3, the key derivation method follows TS 33.501 Annex A.11.

NOTE 1: the order of step 2/3 and step1 follows RAN2 procedure.

Step4. UE performs the inter-CU switch, hanover from gNB1 to gNB2. UE derives K_gNB21 using NCC1.

Step5. After the handover, gNB2 sends NGAP PATH SWITCH COMPLETE message to AMF, including the NCC1 being used.

Step6. AMF sends the NGAP PATH SWITCH ACK message back to gNB2, including a new pair {NH2, NCC2}.

Step7. gNB2 sends RRC message including the NCC2 to UE to be used for next switch.

NOTE 2: which RRC message to be used is based on RAN2 decision.

Step8. gNB2 derives K_gNB12 based on NCC2 and sends it to gNB1, the key derivation method follows TS 33.501 Annex A.11.

Step9. gNB2 derives K_gNB32 based on NCC2 and sends it to gNB3, the key derivation method follows TS 33.501 Annex A.11.

NOTE 3: the order of step 8/9 and step7 follows RAN2 procedure.

Step10. gNB2 sends the MAC CE to the UE to trigger the switch.

Step11. UE performs the inter-CU switch, hanover from gNB2 to gNB3. UE derives K_gNB32 using NCC2.

Step12. After the handover, gNB3 sends NGAP PATH SWITCH COMPLETE message to AMF, including the NCC2 being used.

Step13. AMF sends the NGAP PATH SWITCH ACK message back to gNB3, including a new pair {NH3, NCC3}.

Step14. gNB3 sends RRC message including the NCC3 to UE to be used for next switch.

### 3.X.3	Evaluation

TBD

*** End of 2nd Change ***