3GPP TSG-SA3 Meeting #117	S3-243010

Maastricht, NL, 19 – 23 Aug 2024

# Source:	Nokia

# Title:	New key issue for privacy aspects of MRL

# Document for:	Approval

Agenda Item:	5.17

# 1	Decision/action requested

SA3 is kindly requested to approve the proposed key issue.

# 2	References

# 3	Rationale

This contribution proposes a new key issue for privacy aspects of MRL. 5G NR Femto gets deployed in an untrusted environment. The ongoing study in Rel-19 for security aspects of 5G NR Femto intends to re-use legacy solutions for CAG. If the PNI-NPN solution is readily reused, MRL (Mobility Restriction List) containing the UE Allowed CAG List for the connecting UE will be sent from AMF to the NR Femto. This can pose privacy concerns, considering the fact that 5G NR Femto devices are installed in un-trusted environments. This contribution proposes a new key issue to study this aspects.

# 4	Detailed proposal

******** FIRST CHANGE ********

## 5.X	Key Issue #X: Privacy aspects of MRL

### 5.X.1	Key issue details

Femto devices are deployed at the user locations and considered to be in un-trusted environments. If legacy solutions for CAG are re-used, the MRL (Mobility Restriction List) containing the UE Allowed CAG List for the connecting UE needs to be sent from AMF to the 5G NR Femto. For outbound handovers from a source RAN node to closed NR Femtos, using current PNI-NPN solution means performing access control at the source RAN node i.e. comparing the UE Allowed CAG List with the list of CAG IDs supported by target cell. For inbound handovers from a source RAN node to a target RAN node using current PNI-NPN solution means performing a second access control check at the target RAN node i.e. comparing the UE Allowed CAG List with the list of CAG IDs supported by target cell. In such mobility scenarios, MRL would be required to be shared with 5G NR Femto.

Considering possibility of malicious/compromised femto devices, sending of the UE Allowed CAG List as part of MRL to an 5G NR Femto device could lead to a privacy issues.

### 5.X.2	Security Threats

Considering the fact that femto devices are deployed in un-trusted environments, privacy of UEs can be at risk when MRL is shared with 5G NR Femto.

### 5.X.3	Potential Requirements

The 5G system shall be able to ensure user privacy during mobility involving 5G NR Femto devices.

******** END OF CHANGES ********