3GPP TSG-SA3 Meeting #117	S3-243042

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Ericsson

# Title:	Discussion about Rel-18 ATSSS

# Document for:	Endorsement

Agenda Item:	3

# 1	Decision/action requested

This document provides a discussion about the incoming LS from SA2 regarding the mandatory use of TLS in MP QUIC [1] and proposes to endorse sending a response and creating an Annex in TS33.501.

# 2	References

[1]	S2-2405459/S3-241746 LS on the use of TLS with QUIC in ATSSS context

[2]	SP-240868 On the mandatory use of TLS with MPQUIC in ATSSS context

[3]	SP-241015

[4]	TS 23.501

[5]	TS 33.501

[6]	draft-ietf-quic-multipath-10 - Multipath Extension for QUIC

[7]	S3-242757 Reply LS on Security Considerations for MPQUIC

# 3	Rationale

## 3.1	Related LSs

In SA3#116, SA3 received an LS [1] from SA2, regarding a remaining Editor's note in the Rel-18 specification of ATSSS. There were several discussions during the SA3#116 meeting, but in the end the LS was postponed.

Another LS was submitted for the SA plenary meeting in June of this year [2], where some of the discussions held in SA3#116 were summarized, asking for a way forward. The result of the plenary discussion was that an LS was sent to IETF asking them to expedite the MP QUIC RFC [3] especially regarding the security considerations.

## 3.2	Discussions during SA3#116

### 3.2.1	Server Authentication

For Rel-18 ATSSS there are three ATSSS methods listed in [4], one being the use of MP QUIC, the others are MP TCP and ATSSS LL. The Editor's note mentioned in the LS from SA2 [1] is related to the mandatory use of security in MP QUIC. For MP QUIC, TLS security is used for authentication and encryption between the two peers, in this case being the UE and UPF. The other Rel-18 ATSSS methods does not involve any security mechanisms.

Since MP QUIC is using the same mechanisms as TLS for authentication, the situation is the same as for TLS i.e. server authentication is mandated while client authentication is optional. During the SA3#116 meeting there was some concerns raised and suggestions to study if also client authentication needs to be mandatory in 3GPP. The main arguments against need of UE authentication are:

- In rel-18 ATSSS, MP QUIC is used mainly for its steering possibilities and not as a security protocol. MP QUIC is used on top of existing security measures in 3gpp and non-3gpp access (i.e. UEs are authenticated over non-3gpp access as specified in 33.501 [5]).

- The other variants of Rel-18 ATSSS (MP TCP and ATSSS LL) provide no additional authentication mechanisms and rely on the already existing 3gpp security mechanisms.

Hence there should be no need for mutual authentication in Rel-18 MP QUIC. Since server authentication cannot be excluded, the question about certificate provisioning was also raised in SA3#116.

It was proposed but not agreed to add an Annex to 33.501 describing that UPF certificate can be provisioned during deployment and does not need any normative work.

### 3.2.2	MP QUIC in draft status

The specification of MP QUIC [6] is still in Internet draft status which was mentioned as a potential drawback of the method. The lack of security considerations was also brought up as a potential risk.

Regarding the draft status, MP QUIC internet draft is a working group draft which means that is already has a lot of acceptance and the aim is for it to be an actual RFC.

In the IETF meeting in July 2024, there was further progress on the draft, especially regarding the security considerations [6]. IETF also sent a reply LS to SA plenary [7] explaining that progress was made on the security considerations.

### 3.2.3	UPF exposure to internet

During the SA3#116 discussions, a concern was raised about UPF being exposed to the internet. However, this may be discussed for Rel-19 ATSSS-Lite, it is not relevant for Rel-18 discussion since in that case N3IWF is used between UE and UPF.

# 4	Detailed proposal

It is proposed to add an Annex to TS 33.501 explaining that certificate provisioning in UPF and potential corresponding root certificates used for verification are provisioned as part of deployment and does not need any normative work.

It is also proposed to send a reply LS to SA2 informing SA2 that the Editor's note in TS 23.501 can be removed.