3GPP TSG-SA3 Meeting #117																S3-243049

Maastricht, Netherlands  19 - 23 August 2024 					(revision of S3-yyxxxx)

# Source:	Nokia

# Title:	Discussion paper privacy issue associated with user id

# Document for:	Discussion

Agenda Item:	5.10

# 1	Decision/action requested

It is requested to discuss the discussion paper on Fast BSS solution issue related to NSWO

# 2	References

[X]

# 3	Rationale

Please see details proposal.

# 4	Detailed proposal

For the KI#2 in TR 33.700-32, talks about only limited aspect of privacy issue. In this paper, we are providing additional privacy issue associated with the user Id

### 4.1  Exposure of linked UE subscription information associated with User Identifier:

If user-profile is linked to the UE’s subscription, based on the AF request, the 5GC can expose the linked subscription information, e.g., if User A is registered to 5GC via UE-B, it means User A is using UE-B. Therefore, exposing certain information about the user may lead to a privacy issue of the UE as well. For example,

- User-A is ok to provide the location to AF1 But UE-B is not Ok to provide the location to AF1. Respective data in the UDM/NF contains the respective consent flags. (Step 1,2)

- AF requests NEF and asks for User-A linking (linked subscription) information. And NEF provides the response with User-A is linked with UE-B. (Step 4)

- Then AF requests NEF and asks for the user's location. NEF provides a User-A location (i.e., UE-B location).

- AF determines the UE-B location even if AF is not allowed to get UE-B location information. (Step 5)

Therefore, the following rules would be applied:

- If the AF requests 5GC to provide the user location, then 5GC checks the privacy profile (similar to the consent flag) of UE as defined in TS 23.273 [6] and the consent flag of the user as defined in the user profile (Step 5D). When both UE and the user are allowed to share the location, then only the user’s location should be determined and shared with AF.

### 4.2 Exposure of linked UE subscription information associated with User Identifier to AMF/SMF (internal NFs)

Use case 1: Secondary authentication is enabled:

The user profile contains few IE/properties, like which Slice1/DNN1 (internet) is allowed to the user. Subscription data also contains Subscriber Slice1/DNN1/Slice2/DNN2 information. When the user registers via the UE, then AMF/SMF needs the subscription data to provide the authorization. If AMF/SMF is provided with merge data of subscriber and user, the AMF will receive Slice1/DNN1.

It is possible that user A logs into the UE-B and is able to access enterprise DNN on behalf of subscriber B because secondary authentication is still a legacy method that is based on SUPI. i.e. In Rel19, we don’t have any use case of enhancing the secondary authentication.

Use case 2: Secondary authentication is disabled:

The user profile contains few IE/properties, like which slice1/DNN1 is allowed to the user. Subscription data also contains Subscriber Slice2/DNN2 information. When the user registers via the UE, then AMF/SMF needs the subscription data to provide the authorization. If AMF/SMF is provided with merge data of subscriber and user, the AMF will receive Slice1/DNN1.

It is possible that user-A logs into the UE-B and is able to access enterprise Slice1/DNN1 on behalf of subscriber-.Lets try to understand this with the below figure.

Therefore, the following rules would be applied:

- The UE subscription defines what services are allowed to be used by the user. i.e., if the user can use a certain service, like slice/DNN. Therefore, subscription data contains additional IEs example, Allowed Slice/DNN, Allowed User Id, Allowed Services (e.g. SMS, Call) etc.

- These rules should be applied, and only relevant subscriptions related to User +Subscriber should be provided to AMF/SMF (or any requesting node). So that AMF/SMF can restrict the user accessing the Slice1/DNN1 or allow or reject services.

# 5	Conclusions and proposals

Based on the observations made in this paper, we are proposing to have KI#3 update as defined in S3-243050