3GPP TSG-SA3 Meeting #117	S3-243071

Maastricht, Netherlands 19 - 23 August 2024

# Source:	Huawei, HiSilicon

# Title:	Add evaluation to solution #6

# Document for:	Approval

Agenda Item:	5.15

1	Decision/action requested

Approve the pCR to TR 33.754[1].

2	References

[1]	3GPP TR 33.754: "Study on security aspects for Multi-Access (DualSteer + ATSSS Ph-4) ".

3	Rationale

This contribution proposes to add evaluation to solution #6.

4	Detailed proposal

*** Start of 1st Change ***

6.6	Solution #6: Using IPsec to authenticate UE and UPF for non-3GPP access

6.6.1	Introduction

This solution addresses key issue #1.

The authentication of UE during the IKEv2 procedure can be optimized by using the 3GPP security context for the same UE.

6.6.2	Solution details

In simplified ATSSS architecture, there is an assumption that UE will establish connection with network and establish PDU session using 3GPP access. In that case, before UE connects to UPF through NIN3A, UE and network already shared security context for 3GPP access. So, the authentication method of IKEv2 protocol for non-3GPP access can utilise pre-shared key between UE and UPF instead of EAP-5G method. The specific procedure is as follows:

Figure 6.6.2.1 ATSSS-lite authentication procedure over NIN3A

1. 3GPP registration procedure is executed between UE and network through 3GPP access.

2. UE sends PDU session request to AMF through 3GPP access which carries an ATSSS-Lite indication.

3-4. AMF selects ATSSS-lite enabled SMF and forwards PDU session request to SMF.

5. SMF selects ATSSS-lite enabled UPF and obtains an authentication key KUPF for the direct connection between the UE and the UPF. Then the SMF initiates the N4 Session Establishment procedure with the selected UPF and sends the KUPF to UPF.

6. The UPF allocates the IP address for the IPSec functionality of the UPF and responds to the SMF the N4 Session Establishment Accept message including the UPF IP address which is to be used for establishment of the IPSec tunnel with the UE.

Editor’s Note: The UPF IP address to establish IPsec tunnel, is a dynamic IP address  allocated per UE and PDU session is FFS.

7. UPF IP address is sent to UE along with the PDU session accept message.

8. UE derives the key KUPF used for authentication between UE and UPF.

9. Data Link Layer L2 is connected.

10.The UE starts the IPSec Tunnel Establishment procedure to the UPF indicated by the UPF IP address received in PDU Session Establishment Accept message via 3GPP access, and exchanges the first pair of messages known as IKE_SA_INIT.

11.UE sends the IKE_AUTH_request to the UPF with the Authentication Method “Shared Key Message Integrity Code” (i.e. the value of authentication method is “2”), and the MAC value (MAC1) carried in the AUTH payload is constructed using the derived key KUPF in step 8.

12.The UPF will check the validity of MAC value carried in AUTH payload using the KUPF as described in step 5. If the verification is passed, UPF constructs AUTH payload as in step 11 (with authentication method is “2”, and MAC2 constructed from KUPF) and sends it to UE.

Editor’s Note: The message name is FFS.

UE checks the validity of AUTH payload using the KUPF and if the verification passed the authentication of UPF to UE is also passed.

13. UE and UPF run the CREAT_CHILD_SA procedure and establish the IPsec security tunnel.

6.6.3	Evaluation

This solution addresses key issue #1 and proposes to use IKEv2 protocol to do the authentication during the non-3GPP access procedure for NIN3A scenario.

It requires that the UE and UPF supports the functionality of IKEv2 using a pre-shared key-based authentication method.

UE and UPF have to obtain a pre-shared key before the non-3GPP procedure starts, the key can be derived according to the mechanism defined in solution #1.

*** End of 1st Change ***