3GPP TSG-SA3 Meeting #117	S3-243228

Maastricht, Netherlands 19 - 23 August 2024

# Source:	Samsung

# Title:	Resolving EN in Sol#6

# Document for:	Approval

Agenda Item:	5.2

# 1	Decision/action requested

SA3 is kindly asked to approve this contribution.

# 2	References

[1]	3GPP TR 23.700-77 Study on system architecture for next generation real time communication services Phase 2

# 3	Rationale

Solution#6 has the following Editor's Notes in Solution details.

Editor's Note: Whether the avatar object should be protected is FFS.

Editor's Note: When DAC and XR AS are outside of IMS network, whether it is upto implentation is FFS.

Editor's Note: The alignment with SA2 conclusions from TR 23.700-77 [2] is FFS.

Editor's Note: The details of how the DAC/XR AS verifies the claimed avatar ID based on UE generated attestation are FFS.

Editor's Note: The need for confidentiality protection of the avatar information transportation is FFS.

In case XR AS and BAR is in the same security domain, the protection between them may be upto implementation. In case XR AS and BAR is in the different security domain, the protection between them can be achived via e.g., NEF. So, the protection between XR AS and BAR is out of scope of this solution.

For the second Editor's Note, DCSF needs to check whether the UE-A is allowed to use the claimed avatar ID in the attestation before sending the UE-A's attestation. Therefore, it is not upto implementation.

For the third Editor's Note, according to the conclusion in TR 23.700-77 [1], network/UE based rendering is supported and the avatar ID list is downloaded to the UE through bootstrap data channel via DCSF. As per the conclusion, revision is done in this contribution.

To resolve the fourth Editor's Note, UE-A's certificate is sent to DAC which verifies the claimed avatar ID using the certificate and attestation.

Due the nature of UE-B centric avatar call flow, the avatar object is transferred to UE-B via IMS network which may be different from UE-A's. In this case, UE-A's avatar object, which is privacy related information, needs to be protected against IMS network of UE-B.

# 4	Detailed proposal

*************** Start of 1st Change ****************

## 6.6	Solution #6: Solution for secure IMS based avatar communication

### 6.6.1	Introduction

This solution addresses key issue #2: Security of IMS based Avatar Communication.

According to TR 23.700-77 [2], rendering of avatar media can be performed by network, UE-A, or UE-B, which are called network centric IMS avatar call, UE-A centric IMS avatar call, and UE-B centric avatar call, respectively.

In this solution, only authorized entity from UE-A can access and retrieve the UE-A's avatar object which is stored in a data storage called Base Avatar Repository (BAR). This solution proposes to use UE-A's attestation for the security procedure of IMS based avatar communication.

In this solution, it is assumed that BAR has CA certificate to verify UE-A's certificate, with UE-A's avatar object and avatar ID(s).

The avatar communication can be unidirectional or bidirectional. In this solution, only unidirectional avatar communication is described. When bidirectional avatar communication is used, UE-B also performs the operation same as UE-A described in the procedure.

### 6.6.2	Solution details

#### 6.6.2.1	Network centric IMS avatar call flow

Figure 6.6.2.1-1 Network centric IMS avatar call flow

1.	The UE-A initiates an IMS session and establishes audio and video session connections with the UE-B. The bootstrap channel is established for both the UE-A and UE-B. UE-A obtains UE-A's avatar id(s) through bootstrap data channel via DCSF.

2.	The UE-A performs the XR media rendering negotiation with the XR Application Server. The UE-A generates UE-A's attestation. The UE-A's attestation consists of Avatar ID, identity of the XR AS, rendering option (i.e., network centric), expiration time, signature generated by using UE-A's private key, as described in clause 6.6.2.4 of this document. The UE-A sends UE-A's attestation and UE-A's certificate to the XR Application Server via DCSF. Before sending the attestation, DCSF checks whether the the UE-A is allowed to use the avatar ID.

3.	The UE-A decides to use network centric rendering of an avatar object.

4.	Application data channel is established between UE-A and IMS network, and media re-negotiation may be performed.

5.	XR Application Server requests avatar object using UE-A's attestation and UE-A's certificate.

6.	After BAR verifies the UE-A's certificate, BAR verifies the signature in UE-A's attestation using the UE-A's public key. If the verification is successful and the identity of XR AS in the UE-A's attestation is same as that of the XR Application Server which requested in step 5, the BAR responds with the UE-A's avatar object.

NOTE: Protection between BAR and XR AS is out of scope of this solution.

7.	The XR Application Server sends the avatar object to MF/MRF and requests rendering of the avatar.

8.	The UE-A sends data for the rendering (e.g., facial feature points).

9.	The MF/MRF performs the rendering using the UE-A's avatar object and the data received from UE-A in step 8.

10.	The rendered avatar media is sent as regular video media to UE-B.

11.	The rendered avatar media is sent back to the UE-A as feedback.

#### 6.6.2.2	UE-A centric IMS avatar call flow

Figure 6.6.2.2-1 UE-A centric IMS avatar call flow

1.	The UE-A initiates an IMS session and establishes audio and video session connections with the UE-B. The bootstrap channel is established for both the UE-A and UE-B. UE-A obtains UE-A's avatar id(s) through bootstrap data channel via DCSF.

2.	The UE-A performs the XR media rendering negotiation with the XR Application Server. The UE-A generates UE-A's attestation. The UE-A's attestation consists of Avatar ID, identity of the XR AS, rendering option (i.e., UE-A centric), ephemeral public key of UE-A, expiration time, signature generated by using UE-A's private key, as described in clause 6.6.2.4 of this document. The UE-A sends UE-A's attestation and UE-A's certificate to the XR Application Server via DCSF. Before sending the attestation, DCSF checks whether the the UE-A is allowed to use the avatar ID.

3.	The UE-A decides to use UE-A centric rendering of an avatar object.

4.	Application data channel is established between UE-A and IMS network, and media re-negotiation may be performed.

5.	XR Application Server requests avatar object using UE-A's attestation and UE-A's certificate.

6.	After BAR verifies the UE-A's certificate, BAR verifies the signature in UE-A's attestation using the UE-A's public key. If the verification is successful and the identity of XR AS in the UE-A's attestation is same as that of the XR Application Server which requested in step 5, the BAR generates ephemeral public/private key pair and protects the UE-A's avatar object using the session key generated by ephemeral public key of UE-A included in the UE-A's attestation and ephemeral private key of BAR. The BAR responds with the protected UE-A's avatar object and ephemeral public key of the BAR.

7.	The XR Application Server sends the protected avatar object and ephemeral public key of the BAR to UE-A, and requests rendering of the avatar.

8.	The UE-A generates session key using the ephemeral public key of the BAR and the ephemeral private key of UE-A. UE-A verifies the protected avatar object using the session key and performs the rendering.

9.	The rendered avatar media is sent as regular video media to UE-B.

#### 6.6.2.3	UE-B centric IMS avatar call flow

Figure 6.6.2.3-1 UE-B centric IMS avatar call flow

1.	The UE-A initiates an IMS session and establishes audio and video session connections with the UE-B. The bootstrap channel is established for both the UE-A and UE-B. UE-A obtains UE-A's avatar id(s) through bootstrap data channel via DCSF.

2.	The UE-A performs the XR media rendering negotiation with the XR Application Server and UE-B. In this step, UE-A obtains UE-B's ephemeral public key. The UE-A generates UE-A's attestation. The UE-A's attestation consists of Avatar ID, identity of XR AS, rendering option (i.e., UE-B centric), ephemeral public key of UE-B, expiration time, signature generated by using UE-A's private key, as described in clause 6.6.2.4 of this document. The UE-A sends UE-A's attestation and UE-A's certificate to the XR Application Server via DCSF. Before sending the attestation, DCSF checks whether the the UE-A is allowed to use the avatar ID.

2.	The UE-A decides to use UE-B centric rendering of an avatar object.

4.	Application data channel is established between UE-A and IMS network, and media re-negotiation may be performed.

5.	XR Application Server requests avatar object using UE-A's attestation and UE-A's certificate.

6.	After BAR verifies the UE-A's ceritifcate, BAR verifies the signature in UE-A's attestation using the UE-A's public key. If the verification is successful and the identity of XR AS in the UE-A's atteststion is same as that of the XR Application Server, the BAR generates ephemeral public/private key pair and protects the UE-A's avatar object using the session key generated by ephemeral public key of UE-B included in the UE-A's attestation and ephemeral private key of BAR. The BAR responds with the protected UE-A's avatar object and ephemeral public key of the BAR.

7.	The XR Application Server sends the protected avatar object and ephemeral public key of the BAR to UE-B, and requests rendering of the avatar.

8.	The UE-A sends the data for rendering to UE-B.

9.	The UE-B generates session key using the ephemeral public key of the BAR and the ephemeral private key of UE-B. UE-B verifies the protected avatar object using the session key and performs the rendering using the UE-A's avatar object and the data for rendering received from UE-A in step 8.

#### 6.6.2.4	UE-A attestation

UE-A's attestation is generated by UE-A and it consists as follows:

Table 6.6.2.4-1 UE-A attestation

| Parameter | Description |
| --- | --- |
| Avatar ID | REQUIRED. ID used to retrieve UE-A's avatar object. |
| Identity of XR AS | REQUIRED. The identifier of the XR Application Server that requests UE-A's avatar object to BAR. |
| Rendering option | REQUIRED. One of the followings: network centric, UE-A centric, or UE-B centric |
| Ephemeral public key of UE-A | OPTIONAL. This is included when the rendering option is UE-A centric. BAR uses ephemeral public key of UE-A and ephemeral private key of BAR to protect the UE-A's avatar object. The protected avatar object is sent to UE-A and it is end-to-end protected between BAR and UE-A. |
| Ephemeral public key of UE-B | OPTIONAL. This is included when the rendering option is UE-B centric. BAR uses ephemeral public key of UE-B and ephemeral private key of BAR to protect the UE-A's avatar object. The protected avatar object is sent to UE-B and it is end-to-end protected between BAR and UE-B. |
| Expiration time | REQUIRED. The expiration time of the UE-A attestation. |
| Signature | REQUIRED. Signature generated by UE-A using UE-A's public key and the parameters in the attestation. |

### 6.6.3	Evaluation

TBD