3GPP TSG-SA3 Meeting #117	S3-243256

Maastricht, Netherlands  19 - 23 August 2024

Title:	[Draft] Reply LS on security considerations for 5G ProSe multi-hop relays

Response to:	LS S3-242721/S2-2407207 on LS on security considerations for 5G ProSe multi-hop relays from SA2

Release:	Release 19

Work Item:	FS_5G_ProSe_Ph3_SEC

Source:	Xiaomi [to be SA3]

To:	SA2

Cc:	-

Contact person:	Jouy Shang

shangzhengyi@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

SA3 would like to thank SA2 for their LS on security considerations for 5G ProSe multi-hop relays.

For the support of multi-hop UE-to-Network Relay operations:

Q1: For solution#1, it is assumed that the Intermediate Relay had already been authenticated and established a connection to the network before serving the Remote UE (for discovery or connection). Can SA3 verify the assumption and whether existing UE-to-Network Relay security schemes defined by SA3 in Rel-17 be adapted and reused for this solution (for discovery and connection establishment)?

A1: SA3 would like to confirm this assumption. The existing UE-to-Network Relay security schemes can be adapted and reused for this concluded solution.

Q2: For solution#2 and 7, it is assumed that the Intermediate Relay would not maintain any state of other Intermediate Relays and UE-to-Network Relay, and had no connection established with a UE-to-Network Relay before serving the Remote UE (for discovery or connection). Can existing UE-to-Network Relay security schemes defined by SA3 in Rel-17 be adapted and reused for this solution (for discovery and connection establishment)?

A2: SA3 has identified that the trustworthiness of path information transmitted during the multi-hop UE-to-Network Relay discovery with Model B cannot be guaranteed, if the Intermediate Relay cannot be authenticated and cannot establish a connection with a UE-to-Network Relay before serving the Remote UE.

SA3 suggests to add the same assumption for Model B discovery as for Model A discovery. With this assumption, existing UE-to-Network Relay security schemes can be adapted and reused for the concluded solution.

For the support of multi-hop UE-to-UE Relay operations, no changes to the signalling are expected from the security point of view.

2	Actions

To SA2

ACTION: 	SA3 kindly asks SA2 to take the above information into account.

3	Dates of next TSG SA WG 3 meetings

SA3#118	14 - 18 October 2024		Hyderabad (India)

SA3#119	11 - 15 November 2024		Orlando (US)