3GPP TSG-SA3 Meeting #117	S3-243290

# Maastricht, Netherlands 19 - 23 Aug 2024

# Source:	OPPO

# Title:	Solution on AIoT device ID privacy protection using anonymous key

# Document for:	Approval

Agenda Item:	5.9

1	Decision/action requested

It is proposed to add a sol on AIoT device ID privacy protection using anonymous key.

2	References

N/A

3	Rationale

It is proposed to add a new solution for KI#3 Privacy by protecting AIoT device identifiers of TR 33.713.

4	Detailed proposal

**** Start of Change****

6.Y	Solution #Y: AIoT device ID privacy protection using anonymous key

6.Y.1	Introduction

This solution addresses the security requirement of KI#3: Privacy by protecting AIoT device identifiers.

To prevent the AIoT deivce ID from being exposed in the air interface, the 5GC can trigger the reader(s) to pag an AIoT device or a group of AIoT Devices by broadcasting a partial AIoT Device ID. The AIoT device ID is consist of the common part (e.g., the Home Network identifier, or the 3rd party identifier) and the unique part (e.g., an identifier used to identify a specific Ambient IoT device). The partial AIoT Device ID is the common part of a group of AIoT device ID.

The AIoT deivce matchs the partial AIoT deivce ID will perform random access responding to the pagging message and report its permanent ID to the network. The AIoT device encrypts its permanent ID with an anonymous key AK, which is a shared key between AIoT device and AIoTF. The AK and corresponding Key ID is provided to the Network by the AF. The AIoTF may receive the AK and Key ID from AF’s service request or retrieve such information from UDM (provided in advance by the AF).

6.Y.2	Solution details

This solution describes the AIoT device ID protection as shown in the following figure 6.Y.2-1.

Figure 6.Y.2-1: AIoT device ID privacy protection using anonymous key

- The AF sends the AIoT Service Request to AIoTF, including the AIoT device ID, may include the anonymous key AK and the key ID. The anonymous key AK and the key ID can be provided by the AF to the Network, and pre-stored in UDM. The AK include two kinds of keys: (1) a specific key for each AIoT device; or (2)a group key shared by a group of AIoT devices.

- The AIoTF requests the Reader to start Paging the AIoT device or a group of AIoT devices, using the partial AIoT Device ID.

The AIoT device ID is consist of the common part (e.g., the Home Network identifier, or the 3rd party identifier) and the unique part (e.g., an identifier used to identify a specific Ambient IoT device). The partial AIoT Device ID is the common part of a group of AIoT device ID.

- The Reader pages the AIoT device or a group of AIoT devices using the partial AIoT Device ID.

For one or mautiple AIoT device(s):

- The AIoT deivce matchs the partial AIoT deivce ID will perform random access responding to the message and report its permanent ID to the network. To prevent the AIoT device permanent ID being exposed, the AIoT device encrypts the AIoT device permanent ID with the anonymous key AK. The input parameters to the ciphering should include at least a fresh value, this fresh value changes to prevent the AIoT device from being linked or traced.

- The AIoT device sends the Paging Response to the Reader, including the encrypted AIoT device permanent ID, the Key ID, and the fresh value.

- The Reader sends the encrypted AIoT device permanent ID, the Key ID, and the fresh value to the AIoTF.

- The AIoTF may retrieve the AK using the received Key ID from the UDM if needed. The AIoTF uses the anonymous key AK and the fresh value to decrypt the AIoT device permanent ID.

6.Y.3	Evaluation

This solution fully addresses security requirement in KI#3.

This solution requires the AIoT device to be provisioned with the anonymous key to encrypt the permanent device ID. And this solution is suitable for both Inventory-only case and Inventory and Command case.

**** End of Change****