3GPP TSG-SA3 Meeting #117	S3-243335

Maastricht, Netherlands, 19th – 23rd August 2024

# Source:	Qualcomm Incorporated

# Title:	Updates to solution #10

# Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

This contribution proposes to update the solution #10.

# 2	References

[1]	TR 23.700-03

[2]	TR 33.743

# 3	Rationale

This contribution proposes several updates in solution #10 to align with the SA2 conclusion on the procedure.

First, to align with the solution #1 in TR 23.700-3 [1] which is concluded for model A based discovery and corresponding PC5 link establishment, it is proposed to update that the PC5 link establishment between Intermediate UE-to-Network relay and UE-to-Network relay is performed during multi-hop UE-to-Network Relay discovery with model A procedure. When an Intermediate UE-to-Network relay receives a valid discovery message (i.e., Announcement message in discovery model A) from a UE-to-Network relay, it checks whether there is an existing PC5 link. If there is no existing PC5 link, it initiates the PC5 security establishment for 5G ProSe UE-to-Network relay communication over User Plane.

In addition, the above procedure updates describe how the Intermediate relay decides and triggers PC5 link establishment with the UE-to-Network relay. Therefore, it is proposed to remove the following two Editor’s Notes.

Editor’s Note: How to trigger the PC5 Link establishment with security is FFS.

Editor’s Note: How the Intermediate Relay decides to connect to U2N Relay without connection request from Remote UE is FFS.

Second, the solution assumed one Intermediate UE-to-Network relay in a path. In case there are multiple Intermediate UE-to-Network relays in the path, the PC5 link establishment procedure on each hop link is sequentially performed from the link between the UE-to-Network relay and an Intermediate UE-to-Network relay to the link between an Intermediate UE-to-Network relay and the Remote UE. It is proposed to add a following NOTE to cover the multiple Intermediate UE-to-Network relays scenario:

NOTE x: PC5 link establishment on each hop is sequentially triggered from the link between the 5G ProSe UE-to-Network relay and an Intermediate UE-to-Network Relay to the link between an Intermediate UE-to-Network Relay and Remote UE.

Since the PC5 link establishment procedure with the proposed updates are aligned with the SA2 conclusion, it is proposed to remove the following Editor’s Note.

Editor’s Note: Alignment with SA2 conclusion on the procedures is FFS.

# 4	Detailed proposal

It is proposed that SA3 approved the below changes for inclusion in the draft TR [2].

**** START OF CHANGES ****

## 6.10	Solution #10: Multi-hop UE-to-Network Relay communication security

### 6.10.1	Introduction

This solution addresses the first, second and third security requirements in the key issue #1 regarding the multi-hop UE-to-Network (U2N) Relay communication. This solution proposes to reuse the security procedure over User Plane for PC5 security establishment for each hop among 5G ProSe Remote UE, Intermediate UE-to-Network Relay(s), and 5G ProSe UE-to-Network Relay as specified in clause 6.3.3.2 of TS 33.503 [5]. The proposed security procedure is based on the multi-hop UE-to-Network Relay communication procedures in the solutions (i.e., solution #1) of TR 23.700-03 [1].

### 6.10.2	Solution details

The security procedure for multi-hop UE-to-Network Relay communication is shown in Figure 6.10.2-1.

Figure 6.10.2-1: Security procedure for multi-hop UE-to-Network Relay communication

0.	The 5G ProSe Remote UE, Intermediate UE-to-Network Relay, and 5G ProSe UE-to-Network Relay are provisioned with the discovery security materials associated with an RSC based on the procedure specified in clause 6.3 of TS 33.503 [5]. In addition, the 5G ProSe Remote UE and Intermediate UE-to-Network Relay are provisioned with UP-PRUK and UP-PRUK ID from 5G PKMF as specified in step 1 in clause 6.3.3.2.2 of TS 33.503 [5].

1a.	During multi-hop UE-to-Network Relay discovery with model A procedure, the 5G ProSe UE-to-Network Relay broadcasts an Announcement message.

1b.	If the Intermediate UE-to-Network Relay does not have an existing PC5 link with the 5G ProSe UE-to-Network Relay or an upstream intermediate UE-to-Network relay when it receives a valid discovery message (i.e., Announcement message in discovery model A), the Intermediate UE-to-Network Relay establishes a PC5 link with the 5G ProSe UE-to-Network Relay or the upstream intermediate UE-to-Network relay based on the PC5 security establishment for 5G ProSe UE-to-Network relay communication over User Plane specified in clause 6.3.3.2.2 of TS 33.503 [5].

1c.	Once the PC5 link is established between the Intermediate UE-to-Network Relay and the 5G ProSe UE-to-Network Relay, the Intermediate UE-to-Network updates the path information (e.g., hop count) in the Announcement message and protects the updated message. Then, the Intermediate UE-to-Network Relay broadcasts the protected Announcement message.

2.	After multi-hop UE-to-Network Relay discovery, the 5G ProSe Remote UE establishes a PC5 link with the upstream Intermediate UE-to-Network Relay based on the PC5 security establishment for 5G ProSe UE-to-Network relay communication over User Plane specified in clause 6.3.3.2.2 of TS 33.503[5] with the Intermediate UE-to-Network Relay taking the role of the 5G ProSe UE-to-Network Relay.

NOTE 1: step 3 can start before step 2.

NOTE x: A PC5 link establishment on each hop is sequentially triggered from the link between the 5G ProSe UE-to-Network relay and an Intermediate UE-to-Network Relay to the link between an Intermediate UE-to-Network Relay and Remote UE.

Editor’s Note: How the Intermediate Relay can play the role of U2N Relay to perform UP procedure while out of coverage is FFS.

Editor’s Note: How is the Remote UE authorized for connecting via U2N Relay as per UP procedure is FFS.

Editor’s Note: The need for a Remote UE to establish e2e security with the U2N relay is FFS.

### 6.10.3	Evaluation

TBD

**** END OF CHANGES ****