3GPP TSG-SA3 Meeting #117	S3-243339

Maastricht, Netherlands, 19th – 23rd August 2024

# Source:	Qualcomm Incorporated

# Title:	Add an evaluation to solution #15

# Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

This contribution proposes to add an evaluation to solution #15.

# 2	References

[1]	TR 33.743

# 3	Rationale

This contribution proposes to add an evaluation text to solution #15. Particularly, it is proposed to include that this solution reuses the existing security procedure defined for unicast mode 5G ProSe Direct Communication defined in Rel-17 and is also aligned with the SA2’s conclusion when IP based PDU session type is used for multi-hop UE-to-UE relay.

# 4	Detailed proposal

It is proposed that SA3 approved the below changes for inclusion in the draft TR [1].

**** START OF CHANGES ****

## 6.15	Solution #15: Multi-hop UE-to-UE Relay communication security

### 6.15.1	Introduction

This solution addresses the first, second, and third security requirements in the key issue #2 regarding the multi-hop UE-to-UE (U2U) Relay communication. This solution assumes the architecture and procedures proposed in the solution #3 of TR 23.700-03 [1]. This means that UE-to-UE Relays establish a PC5 link with each other after the Relay discovery to form a 5G ProSe UE-to-UE Relay cloud, and a 5G ProSe End UE establishes a PC5 link with the 5G UE-to-UE Relay to discover a target 5G ProSe End UE and communicate with the discovered target 5G ProSe End UE at IP layer (e.g., based on MANET routing protocol).

For PC5 link security, this solution proposes to reuse the security procedure specified in clause 6.2 of TS 33.503 [5].

### 6.15.2	Solution details

Based on the architecture and procedures in the solution #3 of TR 23.700-3 [1], this solution consists of two types of PC5 link establishment: one for PC5 link establishment among 5G ProSe UE-to-UE Relays and the other one for PC5 link establishment between an 5G ProSe End UE and 5G ProSe UE-to-UE Relay.

1.	PC5 link establishment among 5G ProSe UE-to-UE Relays:

After 5G ProSe UE-to-UE Relays perform a Relay discovery, they establish a secure PC5 link based on the security procedure for unicast mode 5G ProSe Direct Communication specified in clause 6.2 of TS 33.503 [5] with the following modifications:

-	The RSC is included in the DCR message.

-	The DCR message is protected based on the security mechanism defined in clause 6.3.5 with a modification that the UP-PRUK ID/CP-PRUK ID is not used in clause 6.3.5.2.

2. PC5 link establishment between an 5G ProSe End UE and 5G ProSe UE-to-UE Relay:

After the Relay discovery between an 5G ProSe End UE and 5G ProSe UE-to-UE Relay, the 5G ProSe End UE establishes a secure PC5 link with the 5G ProSe UE-to-UE Relay based on the security procedure for unicast mode 5G ProSe Direct Communication specified in clause 6.2 of TS 33.503 [5] with the following modifications:

-	The RSC is included in the DCR message.

-	The DCR message is protected based on the security mechanism defined in clause 6.3.5 with a modification that the UP-PRUK ID/CP-PRUK ID is not used in clause 6.3.5.2.

### 6.15.3	Evaluation

This solution addresses the security requirements of the key issue #2 regarding the multi-hop UE-to-UE (U2U) Relay communication by reusing the security procedure for unicast mode 5G ProSe Direct Communication defined in Rel-17.

This solution is aligned with SA2’s conclusion when IP PDU session type is used for multi-hop U2U Relay.

**** END OF CHANGES ****