3GPP TSG-SA3 Meeting #117	S3-243340

Maastricht, Netherland, 19th - 23rd August 2024

# Source:	Qualcomm Incorporated

# Title:	A new solution for privacy protection of AIoT device identifier

# Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This contribution proposes a new solution to address the key issue #3.

# 2	References

[1]	TR 33.713

# 3	Rationale

This contribution proposes a new solution for addressing the key issue #3: Privacy by protecting AIoT device identifiers. The solution proposes a privacy protection mechanism by using a temporary identifier associated with an AIoT device.

# 4	Detailed proposal

It is proposed that SA3 approved the below changes for inclusion in the draft TR [1].

**** START OF CHANGES ****

## 6.Y	Solution #Y: Privacy protection of AIoT device identifier based on a temporary identifier

### 6.Y.1	Introduction

This solution addressed key issue #3.

This solution provides a privacy protection mechanism by introducing a temporary identifier associated with an AIoT device. The proposed mechanism generates a temporary device identifier (T-ID) when an AIoT device needs to transmit its device identifier towards the network (e.g., for the purpose of Inventory). This solution assumes that the Application Function (AF) manages the AIoT device identifier and the corresponding security protection profile. The security protection profile includes device credential and an algorithm to use to generate a T-ID.

NOTE 1: The privacy protection mechanism based on a temporary identifier is applied when the AIoT device and AF are provisioned with the security protection profile.

NOTE 2: Device credential types are determined based on each AIoT service and device capability.

### 6.Y.2	Solution details

#### 6.Y.2.1	Procedures

Figure 6.Y.2.1-1: Privacy protection based on a temporary device identifier (T-ID) during Inventory procedure

NOTE 3: The reference architecture and Inventory procedure described in clause 6.3 of TR 23.700-13 (i.e., solution #3) [4] are used to describe the privacy protection mechanism in this solution. The proposed mechanism can be applied to any reference architectures and procedures for AIoT services that require transmission of AIoT device identifier by AIoT device.

0.	Each AIoT device is provisioned with its AIoT device identifier and security protection profile such as a device credential and an algorithm to use to generate a T-ID. An Application Function (AF) manages the AIoT device identifier and the associated security protection profile.

1-3.	The AF triggers an Inventory procedure towards AIoT devices.

4.	Upon receiving the Inventory Request from the Reader, the AIoT device replies to the Reader with its temporary device identifier (T-ID). The AIoT device generates a new T-ID as described in clause 6.Y.2.2.

5.	Upon receiving a T-ID, the Reader sends the received T-ID to the AIoT Controller. Additionally, the Reader sends Enrichment data (e.g., the Reader location) along with the T-ID if configured by the AIoT Controller.

6.	The AIoT Controller stores the received T-ID and Enrichment data.

7.	The AIoT Controller provides the T-ID(s) and the Enrichment data to the AF if T-ID includes the service identifier (e.g., AF Identity). Otherwise, the AIoT Controller provides the information when it receives a Data Request from the AF (7a in Figure 6.Y.2.1-1). In this case, the AF generates the list of T-IDs that are interested in and includes them in the Data Request.

#### 6.Y.2.2	Generation of a temporary identifier

A new temporary device identifier (T-ID) is generated as follows:

T-ID = F(K, freshness parameter, AIoT device identifier), where F is a service specific function that generates a temporary ID. The K is the key provisioned at the AIoT device for temporary ID generation. The freshness parameter is determined based on the device capability. For example, an index can be used as a freshness parameter along with a refresh timer. In this case, the AIoT device increments the index if a refresh timer has expired.

### 6.Y.3	Evaluation

This solution addresses the security requirements in KI #3 by introducing a temporary identity based on symmetric key provisioned at an AIoT device and AF.

**** END OF CHANGES ****