3GPP TSG-SA3 Meeting #117	S3-243351

Maastricht, Netherlands 19 - 23 August 2024

# Source:	Samsung

# Title:	New solution for protecting AIoT ID

# Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

SA3 is kindly asked to approve this contribution.

# 2	References

NA

# 3	Rationale

It is proposed to address key issue #3: Privacy by protecting AIoT device identifiers.

# 4	Detailed proposal

*************** Start of 1st Change ****************

## 6.X	Solution #X: Solution for protecting AIoT ID by using temporary ID

### 6.X.1	Introduction

This solution addresses key issue #3: Privacy by protecting AIoT device identifiers.

According to TR 23.700-13 [4], DO and DO-DTT traffic types for AIoT (Ambient IoT) device are to be considered. This solution proposes to protect AIoT ID in this traffic types.

It is assumed that AIoT device and AF are provisioned with AIoT device identifier and a permanent key (K).

### 6.X.2	Solution details

Figure 6.X.2-1 AIoT ID protection call flow

0. AIoT ID and a permanent key (K) are provisioned to the AIoT and AF. AF generates KAIoT from K and RANDAF.

1. AF sends AIoT service request message to AIoTF (AIoT Function). This message may be sent via NEF. AIoT service request message includes Temp AIoT ID #1, RANDAF, and KAIoT.

NOTE: If this message is sent for the first time, the Temp AIoT ID #1 is the AIoT ID.

2. AIoTF transfers AIoT service request message. This message includes RANDAIoTF, RANDAF, and Temp AIoT ID #1.

3. AIoT generates MAC using RANDAIoTF and KAIoT.

4. AIoT responds with AIoT service response. The message RANDAIoT, Temp AIoT ID #1, and MAC.

5. After AIoTF finds KAIoT from Temp AIoT ID #1 received in step 4, AIoTF checks the authenticity of the message by verifying the MAC.

6. If the verification in step 5 is successful, AIoTF sends AIoT service response to AF. Temp AIoT ID #1 is included in this message.

7. AIoT and AF generates Temp AIoT ID #2 from KAIoT, RANDAIoT, and Temp AIoT ID #1. The Temp AIoT ID #2 is used next time the AF requests a service to AIoT.

### 6.X.3	Evaluation

TBD