3GPP TSG-SA3 Meeting #117	S3-243437

Maastricht, Netherlands, 19 - 23 August 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.117 | CR | draftCR | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test Case on No Web Server Error Pages Info |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, BSI (DE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_Maint |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The GSMA NESAS has reviewed and proposed changes to the existing test case via the document S3-234423. The proposed changes in this document are with reference to Slide#40. Merged with BSI (DE) CR S3-242974 The current test description does not specify exactly how and what to test for. There are a lot of different http error codes (e.g. 20+ in the 4xx class) that could potentially result in a custom error page or error message. The vendor should provide documentation on how he handles custom error pages and messages and how to trigger those. |  |  |  |  |  |  |  |  |
| Summary of change: |  | GSMA suggested changes: Define what is or is not to be used as an automatic assessment tool. Which error message/pages are to be checked? As for other similar tests, these are repeating the requirement, rather than specifying the steps of the testing that is required for the test. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test results and misinterpretation of test execution steps. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.4.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Initial draft and presentation. |  |  |  |  |  |  |  |  |

******************** Start of 1st Change ************************************************************

#### 4.3.4.12	Web server information in error pages

Requirement Name: Web server information in error pages.

Requirement Reference: In accordance with industry best practice

Requirement Description: User-defined error pages shall not include version information about the web server and the modules/add-ons used. Error messages shall not include internal information such as internal server names, error codes, etc. Default error pages of the web server shall be replaced by error pages defined by the vendor.

Threat References: TR 33.926 [4], 5.3.6.5, System Fingerprinting

Test Case:

Test Name: TC_NO_WEB_SERVER_ERROR_PAGES_INFORMATION

Purpose:

To verify that error pages and error messages do not include information about the web server.

Procedure and execution steps

Pre-Conditions:

-	The tester has needed administrative privileges.

-	A tester machine is available.

- 	The vendor provides documentation on user-defined error pages (e.g. location, content, where configured) and messages.

-	The vendor provides a list of potential parameters/commands to trigger events resulting in an http status code 3xx, 4xx, 5xx.

-	The tester should have configured a script, or an automatic assessment tool adapted in line with the Requirement Description.

Execution Steps

1.	The tester verifies that the web server configuration does replace default error pages with error pages defined by the vendor.

2.	The tester verifies that the vendor defined error pages do not contain information about the web server.

3.	The tester triggers and captures at least one occurrence of the following HTTP status code classes:

a) 	Redirection error response (300-399)

b)	Client error response (400-499)

c)	Server error response (500-599)

NOTE 1: 	Possible error pages that could be displayed are: 3xx: redirection, 4xx: client errors, 5xx: server errors.

NOTE 2: 	The 3xx error pages could be triggered by permanent or temporary move of content to other URL and the page is found because redirected.

NOTE 3: 	The 4xx error page could be triggered by trying to access a URL pointing to a non-existent or restricted resource.

NOTE 4: 	The 5xx error page could be triggered by requesting a HTTP method the web server does not support or disabled (e.g. CONNECT, PUT, PATCH).

Expected Results:

-	Generated error pages and error messages do not include information about the web server.

Expected format of evidence:

-	Log files and screen shots of test executions

******************** End of 1st Change ************************************************************