3GPP TSG-SA3 Meeting #117	S3-243454

Maastricht, Netherlands  19 - 23 August 2024

# Source:	Huawei, HiSilicon

# Title:	Sol#19 update to address ENs

# Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

Approve the pCR proposal for TR 33.700-32

# 2	References

[1]	3GPP TR 33.700-32

# 3	Rationale

This contribution proposes text to address 3 ENs for Sol#19 in TR 33.700-32 [1].

# 4	Detailed proposal

pCR

***  BEGINNING OF CHANGES  ***

## 6.19	Solution #19: User privacy protection

### 6.19.1	Introduction

This solution addresses the key issue #2.

The solution protects a user identifier (UID) used in 3GPP system by replacing it with the concealed version, i.e. user identifier concealed (UIC). Since UID is not in the realm of 3GPP systems, the mapping between the UID and UIC is out of scope of 3GPP system, i.e. UIC and how UID is mapped to UIC is transparent to the 3GPP systems.

In addition, the mapping between UIC and UID is not fixed and changed from time to time to avoid the user being tracked or linked.

NOTE 1: the UID is concealed by the UE, e.g., through hashing or a mapping table. The AAA knows the mapping between the UID and its concealed version UIC. The 3GPP system is provisioned with the UIC and the linkage between the UIC and the subscription.

NOTE 2: the UIC can be included in the PDU session request by the UE. It is not necessary for the 3GPP system to know the UID.

### 6.19.2	Solution details

A user authentication procedure is exemplified below to illustrate how to use UIC, instead of UID to protect user privacy. It is notable that the authentication procedure is made generic as much as possible to accommodate different authentication methods (this solution is not meant to address authentication procedure).

The exemplified user identity authentication and authorization (UIAA) procedure with user privacy protection is as follows:

1.The UE sends a Registration Request to the AMF.

2. The AMF triggers the UIAA procedure.

3: The AMF sends the AAA-Server the UIAA message with GPSI included.

NOTE: The UID (but not UIC) is transparent to the AMF/UDM. The UIC and the linkage between the UIC and the UE subscription is provisioned in the AMF/UDM.

4. The AAA-S and the UE perform user authentication and authorization with respect to the user, e.g. the user is identified as UIC between AAA-S and the UE. However, the UIC is transparent to the AMF. There can be multiple message exchanges between UE and AAA-S, depending on authentication method used. The authentication method is out of scope of 3GPP.

5. The AAA-S sends the authentication results to AMF, which stores the result identifying the user by UIC, the concealed version of UID. The UIC should be changed for every authentication to avoid being tracked or linked to previous authentication.

6. The AMF sends registration accept to the UE.

Figure 6.19.2-1 illustration of user privacy protection in a user authentication procedure

### 6.19.3	Evaluation

***  END OF CHANGES  ***