3GPP TSG-SA3 Meeting #117	S3-243490-r1

Maastricht, Netherlands 19 - 23 August 2024

# Source:	Huawei, HiSilicon

# Title:	overall evaluation for KI#1

# Document for:	Approval

Agenda Item:	5.4

# 1	Decision/action requested

Approve to include pCR proposal for the TR 33.776

# 2	References

[1]	3GPP TR 23.776 	Study of Automatic Certificate Management Environment (ACME) for the Service Based Architecture (SBA)

# 3	Rationale

The contribution provides evaluation for all solutions that addressing KI#1, i.e., Sol #1 - Sol #3, in TR 33.776 [1].

# 4	Detailed proposal

pCR

***  BEGINNING OF CHANGES ***

## 7	Conclusions

### 7.1	Overall evaluation for KI#1

There are three solutions addressing KI#1 on the ACME initial trust framework, i.e., Sol#1-Sol#3. In this clause, comparison is provided to evaluate these solutions aganst the existing initial trust setup framework specified in the clause 10.2.2 of the TS 33.310 [3] as shown in the table below.

|  | Initial trust framework (How to establish trust between NF and operator CA) | More details |
| --- | --- | --- |
| TS 33.310 [3] | *OAM has preestablished trust with operator CA *OAM facilitates establishment of initial trust between NF and operator CA/RA | OAM issues NF a) a certificate, or b) Initial Authentication Key (IAK), or c) signature of certain NF profile parameters, including NF instance ID. |
| Solution #1 | *OAM has preestablished trust with DNS server. *DNS server (+ACME procedures) facilitates establishment of initial trust between NF and operator CA/RA (RFC 8555 [2]) | ACME procedures a) DNS: trusted mapping of DN and IP address b) ACME: verifiable control of DN |
| Solution #2 | Same framework as in TS 33.310 [3] | Support option (c) Only |
| Solution #3 | Same framework as in TS 33.310 [3] | Support all 3 options |

Both Sol#2 and Sol#3 reuse the initial trust schema defined in clause 10.2.2 of TS 33.310 [3]. Sol#3 supports all the mechanism options specified in TS 33.310 [3] to establish the initial trust. Sol#2 supports only one option, i.e., the optoin (c), but does not support using the ACME procedure to establish initial trust with the options a) and b).

Sol#1 is not based on the initial trust schema in TS 33.310 [3]. Instead, it relies on a pre-established trust between the OAM and the DNS server, as in RFC 8555 [2]. The ACME procedure is used to establish the trust between the NF and the operator CA by NF providing verifiable control of DN.

***   END OF CHANGES   ***