3GPP TSG-SA3 Meeting #117	S3-243502

Maastricht, Netherlands, 19th – 23rd August 2024

# Source:	MITRE-FFRDC

# Title:	New solution for KI #1 Data Collection using DCCF

# Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

This pCR proposes a new solution #X in TR 33.794[1]

# 2	References

[1]	3GPP TR 33.794 Study on enablers for Zero Trust Security

[2]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

# 4	Rationale

This pCR proposes a new solution #x in 3GPP TR 33.794[1] reusing concepts, called framework in the contribution, from 3GPP TS 23.288 [2] and the SDPI from solution #3. It could be considered a solution #3 alternative but for ease of documentation it is described as a new solution.

## 4.1	Reusing existing SBA concepts

### 4.1.1	Background

Key Issue 1 is basically addressing the following aspects

- Need to inform NFs to collect analytics data

- Once data has been collected deliver the data to an OSF or store for later.

When designing a solution, it makes sense to check whether there is any procedure set already specified by 3GPP and can it be adapted to perform the task at hand.  If it can indeed be adapted to perform the task, then one must also ask whether that introduces any issues.

### 4.1.2	Discussion

Two functions, DCCF and NWDAF (specified in 3GPP TS 23.288 [2] – See Table 2) in the 5G SBA have been defined to collect data from network entities and deliver it to various entities.

## 5A.1 General Data Collection Coordination and Delivery coordinates the collection and distribution of data requested by NF consumers. It prevents data sources from having to handle multiple subscriptions for the same data and send multiple notifications containing the same information due to uncoordinated requests from data consumers. Data Collection Coordination and Delivery is supported by a DCCF via Ndccf_DataManagement service or by an NWDAF via Nnwdaf_DataManagement service. Unless otherwise stated, capabilities specified in clause 5A for a DCCF are also applicable to an NWDAF.

Table 2 – Data collection functions 3GPP TS 23.288 [2]

NOTE:	As illustrated in Table 2 there are 2 functions that can perform data collection, DCCF and NWDAF.

These 2 entities can collect Analytics data or Event Data.  Event Data is qualified by an Event ID defined in 3GPP TS 23.502 [x] sub-clause 4.15.3.1. Analytics data is qualified by Analytics ID specified in GPP TS 23.288 [2] Table 7.1-2.

Another aspect that 3GPP TS 23.288 [2] bolsters is the ability of 3GPP TS 23.288 [2] NFs and other entities (e.g. data consumer) to discover the capabilities of other NFs via the NRF.  This is of particular interest as it allows the OSF, or a 3GPP entity acting on behalf of the OSF, to determine what NF instance IDs or NF types can support SDPI functionality.

## 5A.2 Data Collection Coordination Data Collection Coordination is supported by a DCCF or an NWDAF. The Data Consumer may use an NRF to perform NF discovery and selection to find a DCCF that can coordinate data collection (DCCF discovery principles are defined in clause 6.3.19 of TS 23.501 [2]). Data Consumers send requests for data to the DCCF rather than directly to the NF Data Source. Whether the data consumers directly contact the NF Data Source or goes via the DCCF is based on configuration of the data consumers. For the Data Consumer and each notification endpoint in a data request, the Data Consumer may specify Formatting and Processing Instructions that determine how the data is to be provided. Upon receiving a request from a Data Consumer, the selected DCCF determines the NF instance that can be a Data Source if the Data Source is not indicated in the Data Consumer's request. The DCCF may also select an ADRF if the data is to be stored in an ADRF and an ADRF endpoint is not indicated in the Data Consumer's request

Table 3 – Using NRF to find out capabilities 3GPP TS 23.288 [2]

The following section from 3GPP TS 23.288 [2] (see Table 4) provides an example of how Event data can be collected, where the Data Consumer is the OSF (rather than the NWDAF as stated).  Another example in 3GPP TS 23.288 [2] sub-clause 6.1.4.2 shows where analytics data is consumed, where the Analytics consumer can equally be the OSF.

##### 6.2.6.3.2 Data Collection via DCCF The procedure depicted in Figure 6.2.6.3.2-1 is used by a data consumer (e.g. NWDAF) to obtain data and be notified of events via the DCCF using Ndccf_DataManagement_Subscribe service operation. Whether the data consumer directly contacts the Data Source or goes via the DCCF is based on configuration of the data consumer. Figure 6.2.6.3.2-1: Data Collection via DCCF

Table 4 – Example Data Consumer (e.g. OSF) obtaining data 3GPP TS 23.288 [2]

The above is just one diagram, however 3GPP TS 23.288 [2] describes a framework that among other things also includes how to identify NFs (e.g. NF instance ID, NF type), when data should be collected and where it should be stored e.g. ADRF.

### 4.1.3	Conclusion

3GPP TS 23.288 [2] provides a framework for collecting data (Analytics or Event) from NFs, storing it and delivering it to a analytic or data consumer e.g. OSF.

One issue with using 3GPP TS 23.288 [2] is the potential mixing of data being collecting to be used for security purposes and that for other non-security operations.  It is proposed that this should be a deployment choice, in that it is possible to deploy a DCCF /ADRF that only implements security aspects and another DCCF/ ADRF for other network management functions, these NFs can even use their own different physical transport networks, however these and MNO decisions and outside scope of 3GPP.  3GPP TS 23.288 [2] is purely a vehicle used for documenting capabilities without duplication of effort and an operator can choose how to deploy the functionality.

# 4	Detailed proposal

SA3 is kindly requested to approve the below change to TR 33.794 [1]

-

****Start of 1st   CHANGE ****

## 7.X	Solution #x: Data Collection using DCCF

### 7.X.1	Introduction

This solution addresses KI#1 Data exposure for security evaluation and monitoring. Specifically, it addresses the requirement: (2) Architecture to be used for exposure of data collected for security evaluation and monitoring of the 5G SBA.

This solution defines the security architecture elements that function as the Security Data Points of Ingest (SDPIs) which can be used by the operator’s security function (OSF) to perform the role of policy decision points (PDPs). SDPIs, in this proposal, are logical entities that enable NFs to provide security data (see sub-clause 7.X.2.4). When an SDPI is implemented as

a)	a sidecar on the NF, the SDPIs can read all SBI security data on the NF. SDPI then registers with an NRF as described in clause 7.3;

b)	a subfunction of an NF, the NF profile is updated to include that the NF can provide security data.

NOTE:	In b) it is assumed that the NF providing the security data is secure.  This is an operational / deployment decision.

Security data is to be sent securely using the framework described in 3GPP TS 23.288 [13] which then delivers this data to the OSF.

The OSF remains outside the scope of 3GPP but is expected to provide the logic of PDP and takes the role of a "Data Consumer" as defined in 3GPP TS 23.288 [13].

Editor’s Note: how to secure the communication between the NF and OSF when the OSF is out of 3GPP scope.

Editor’s Note: How to generate the logic of PDP by the OSF if the OSF is out of scope of 3GPP.  The format and content is FFS.

### 7.X.2	Solution details

#### 7.X.2.1	NF profile updates

When an NF supports SDPI functionality, an NFs profile  can be updated to indicate support for SDPI and what security data that SDPI is configured to collect.  The NF  can register this information in the NRF per 3GPP TS 23.501 [13].

The DCCF profile may need updating to indicate support for collecting security data.

#### 7.X.2.2	Data Collection Configuration

Figure 7.X.2.2-1: Data analytics information request

1. The OSF (i.e. Data Consumer) sends a Ndccf_DataManagment_Subscribe message requesting security data (see sub-clause 7.X.2.4).

2a/b/c	The 3GPP TS 23.288 [13] framework / DCCF sends messages to NFs (i.e. Data sources) (e.g. see 3GPP TS 23.288 [13] sub-clause 6.2.6.3.2) requesting security data (see sub-clause 7.X.2.4).

Editor’s Note: Whether the DCCF can be used for security event evaluation and its impacts need to be confirmed by SA2.

Editor’s Note: How does the OSF as a non3GPP NF know which data needs to be collected is FFS.

NOTE:	3GPP TS 23.228 [13] allows for OA&M data (e.g. security logs, counters etc) to be collected, see 3GPP TS 23.288 [13] sub-clause 6.2.6.3.2 step 8a.

#### 7.X.2.3	Data delivery

Figure 7.X.2.3-1: Data transfer

1a/b/c.	3GPP TS 23.288 [13] framework receives security data (e.g. subscribed event data) (see sub-clause 7.X.2.4) from an NF SDPI (e.g. NF1, Nf2, Nf3) ("Data Sources") using functionality specified in 3GPP TS 23.288 [13] (e.g. sub-clause 6.2.6.3).

2	3GPP TS 23.288 [13] framework DCCF decides, based on functionality specified in 3GPP TS 23.288 [13] (e.g. sub-clause 6.2.6.3), sends security data it has received to the OSF (i.e. Data Consumer).

#### 7.X.2.4	Security data

Security data is the data that an SDPI collects and sends to the OSF. The OSF may provide formatting and/or processing instructions to the 3GPP TS 23.288 [13] framework.

Security data that can be collected from an SDPI is specified in the conclusions section of this document.

Editor’s Note: The formatting and the content of the instructions are FFS.

### 7.X.3	Evaluation

Editor’s Note:

Evaluation is FFS

****End of 2nd  CHANGE ****

The Green, yellow and cyan coloring are to help the reader see the relational why the statements where made based on extracts from existing standards.