3GPP TSG-SA3 Meeting #117	S3-243526

Maastricht, the Netherlands, 19th - 23th August 2024

# Source:	ZTE Corporation

# Title:	NCC synchronization solution for LTM

# Document for:	Approval

Agenda Item:	4.12

1	Decision/action requested

This contribution proposes a new solution for LTM.

2	References

[1]	S3-241773 (R2-2404037) LS on security handling for inter-CU LTM in non-DC cases

3	Rationale

Since the MAC CE is currently not protected, the NCC value can be tampered by the attack, which leads to the re-keying between the UE and the gNB out of synchronization. This contribution proposes a new solution for LTM by enhancing the option 1A in [1] to solve the re-keying synchronization issue.

4	Detailed proposal

**** Start of Changes****

3.Y	Solution #Y: NCC synchronization solution for LTM

3.Y.1	Introduction

This solution is for key issue #X. This solution is an enhancement of option 1A where the NCC value is included in the cell switch command MAC CE.

The principle of the solution is :

Upon receiving the NCC value in the cell switch command, the UE compares the received NCC value with the locally stored one.

- If the received NCC value is smaller than the locally kept value, the UE discards the MAC CE and initiates the RRC re-establishment procedure since the message may be tampered.

- If the received NCC value is larger than the locally kept value, the UE synchronizes the NH parameter and computes the KNG-RAN* as described in the solution details.

- If the received NCC value is the same as the locally kept value, the UE computes the KNG-RAN* directly using the current parameters.

3.Y.2	Solution details

Figure 3.Y.2-1. Signalling procedure for inter-CU LTM

- The UE sends a MeasurementReport message to the gNB. The gNB decides to configure LTM and initiates LTM preparation.

- The source gNB issues a Handover Request message to one or more candidate cells belonging to one or more candidate gNBs. The Handover Request passing a transparent RRC container with necessary information to prepare the handover at the target side. Keys for the target gNBs are not derived and not sent to the target gNB.

- Admission Control may be performed by the target gNB.

- The target gNB prepares the handover with L1/L2 and sends the HANDOVER REQUEST ACKNOWLEDGE to the source gNB, which includes a transparent container to be sent to the UE as an RRC message to perform the handover.

- The gNB transmits an RRCReconfiguration message to the UE including the LTM candidate configurations. The gNB also includes the NCC into the message.

- The UE stores the LTM candidate configurations and correspongding NCC, and transmits an RRCReconfigurationComplete message to the gNB.

- The UE performs DL and UL synchronization with the LTM candidate cells as existing procedure.

- 7.	The UE performs L1 measurements on the configured candidate cell(s) and transmits L1 measurement reports to the gNB. L1 measurement should be performed as long as RRC reconfiguration (step 4) is applicable.

- 8.	The gNB decides to execute LTM. The gNB performs vertical key derivation and forward the { KNG-RAN*, NCC} pair to the target. The target gNB/ng-eNB uses the received KNG-RAN* directly as KgNB to be used with the UE. The target gNB/ng-eNB associates the NCC value received from source gNB/ng-eNB with the KgNB.

- 9.	The target gNB sends the Key Update Acknowledge to the source gNB.

10.	The gNB transmits a MAC CE triggering cell switch by including the candidate configuration index of the target cell. The gNB also includes the NCC in plain text into the message. As soon as the UE receives the NCC, it compares the the NCC value with its own NCC (or the NCC value received in Step 4). If it is smaller than the NCC in UE (or the NCC received in Step 4), it indicates that the message may be tampered with and handover is canceled. Then the UE initiates the RRC re-establishment procedure. If the UE received an NCC value that was larger than the NCC associated with the currently active KgNB, the UE shall first synchronize the locally kept NH parameter iteratively and increasing the NCC value until it matches the NCC. When the NCC values match, the UE computes the KNG-RAN* from the synchronized NH parameter. if the UE received an NCC value that was the same as the NCC associated with the currently active KgNB, the UE computes the KNG-RAN* directly using the current parameters. The UE switches to the target cell and applies the configuration indicated by candidate configuration index.

11.	The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell.

12.	The UE completes the LTM cell switch procedure by sending RRCReconfigurationComplete message to target cell. If the UE has performed a RA procedure in step 11 the UE considers that LTM cell switch execution is successfully completed when the random access procedure is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the network has successfully received its first UL data.

13.	Path switch procedure between the target gNB, AMF and UPF.

The steps 6-13 can be performed multiple times for subsequent LTM using the LTM candidate configuration(s) provided in step 4.

Editor’s Note: How to solve the forward security is FFS.

Editor’s Note: How to protect NCC is FFS.

Editor’s Note: How to support AS security context synchronization after path switch is FFS.

Editor’s Note: How to deal with the received NCC value smaller than the locally kept one is FFS.

3.Y.3	Evaluation

TBD

**** End of Changes****