3GPP TSG-SA3 Meeting #117	S3-243570

Maastricht, Netherlands, 19th – 23rd August 2024

# Source:	Qualcomm Incorporated

# Title:	Addressing ENs in solution #9

# Document for:	Approval

Agenda Item:	5.12

# 1	Decision/action requested

This contribution proposes to address the Editor’s Notes in solution #9.

# 2	References

[1]	TR 33.743

# 3	Rationale

This contribution proposes to remove the two Editor’s Notes.

First, Remote UE, Intermediate U2N relay and U2N relay reuse the existing procedure as specified in clause 6.3 of TS 33.503 for discovery security materials provisioning. This means Remote UE, Intermediate U2N relay, and U2N Relay are provisioned with discovery security materials from the 5GPKMF/5G DDNMF of their HPLMN. Thus, it is proposed to update the description of step 0 to clarify that the discovery security materials are provisioned from 5G PKMF/5G DDNMF of Remote UE/Intermediate U2N relay/U2N relay’s HPLMN, and remove the following Editor’s Note.

Editor’s Note: Which HPLMN provision the discovery security materials associated with RSC to the Remote UE, Intermediate Relay and UE-to-Network Relay is FFS.

Second, a path information (e.g., hop count) in the discovery is integrity protected based on the provisioned DUIK to ensure that an unauthorized attacker cannot manipulate the information. Therefore, it is proposed to remove the following Editor’s Note.

Editor’s Note: How the solution protects the path information during the discovery of multi-hop U2N relay is FFS.

# 4	Detailed proposal

It is proposed that SA3 approved the below changes for inclusion in the draft TR [1].

**** START OF CHANGES ****

## 6.9	Solution #9: Multi-hop UE-to-Network Relay discovery security

### 6.9.1	Introduction

This solution addresses the first, second and fourth security requirements in the key issue #1 regarding the multi-hop UE-to-Network (U2N) Relay discovery. This solution proposes to reuse the security procedure for 5G ProSe UE-to-Network Relay discovery with Model A and Model B as specified in clause 6.3 of TS 33.503 [5]. In addition, it is proposed to mandate the integrity protection of discovery messages as the messages contain information related to path selection (e.g., hop count indicating the number of hops to reach the 5G ProSe UE-to-Network Relay). The proposed security procedure is based on the multi-hop UE-to-Network Relay discovery procedures in several solutions (e.g., solution #1, #2, and #7) of TR 23.700-03 [1].

### 6.9.2	Solution details

#### 6.9.2.1	Discovery with Model A

The security procedure for multi-hop UE-to-Network Relay discovery with Model A is shown in Figure 6.9.2.1-1.

Figure 6.9.2.1-1: Model A Discovery operation supporting multi-hop UE-to-Network Relay

0.	The 5G ProSe Remote UE/Intermediate UE-to-Network Relay/5G ProSe UE-to-Network Relay is provisioned with the discovery security materials associated with an RSC from the 5G PKMF/5G DDNMF of 5G ProSe Remote UE/Intermediate UE-to-Network Relay/5G ProSe UE-to-Network Relay’s HPLMN based on the procedure specified in clause 6.3 of TS 33.503 [5]. The discovery security materials contain a Discovery User Integrity Key (DUIK) for the integrity protection of Relay Discovery Announcement.

1.	The 5G ProSe UE-to-Network Relay protects a Relay Discovery Announcement using the discovery security materials associated with the RSC as specified in clause 6.3 of TS 33.503 [5]. Then, the 5G ProSe UE-to-Network Relay broadcasts the Relay Discovery Announcement.

2.	The Intermediate UE-to-Network Relay processes the received Relay Discovery Announcement message using the discovery security materials associated with the RSC as specified in clause 6.3 of TS 33.503 [5]. If the processing is successful, the Intermediate UE-to-Network Relay updates the path information (e.g., hop count, Relay Info.) and protects the updated message using the discovery security materials associated with the RSC as specified in clause 6.3 of TS 33.503 [5]. Then, the Intermediate UE-to-Network Relay broadcasts the message.

3.	Upon receiving the Relay Discovery Announcement message from the Intermediate UE-to-Network Relay, the 5G ProSe Remote UE processes the received message using the discovery security materials associated with the RSC as specified in clause 6.3 of TS 33.503 [5].

#### 6.9.2.2	Discovery with Model B

The security procedure for multi-hop UE-to-Network Relay discovery with Model B is shown in Figure 6.9.2.2-1.

Figure 6.9.2.2-1: Model B Discovery operation supporting multi-hop UE-to-Network Relay

0.	The 5G ProSe Remote UE/Intermediate UE-to-Network Relay/5G ProSe UE-to-Network Relay is provisioned with the discovery security materials associated with an RSC from the 5G PKMF/5G DDNMF of 5G ProSe Remote UE/Intermediate UE-to-Network Relay/5G ProSe UE-to-Network Relay’s HPLMN based on the procedure specified in clause 6.3 of TS 33.503 [5]. The discovery security materials contain a Discovery User Integrity Key (DUIK) for the integrity protection of Relay Discovery Solicitation and Relay Discovery Response.

1.	The 5G ProSe Remote UE protects a Relay Discovery Solicitation using the discovery security materials associated with the RSC as specified in clause 6.3 of TS 33.503 [5]. Then, the 5G ProSe Remote UE broadcasts the Relay Discovery Solicitation.

2.	The Intermediate UE-to-Network Relay processes the received Relay Discovery Solicitation using the discovery security materials associated with the RSC as specified in clause 6.3 of TS 33.503 [5]. If the processing is successful, the Intermediate UE-to-Network Relay updates the path information (e.g., hop count) and protects the updated message using the discovery security materials associated with the RSC as specified in clause 6.3 of TS 33.503 [5]. Then, the Intermediate UE-to-Network Relay broadcasts the message.

3.	Upon receiving the Relay Discovery Solicitation from the Intermediate UE-to-Network Relay, the 5G ProSe UE-to-Network Relay processes the received message using the discovery security materials associated with the RSC as specified in clause 6.3 of TS 33.503 [5]. If the processing is successful, the 5G ProSe UE-to-Network Relay constructs a Relay Discovery Response and protects it using the discovery security materials associated with the RSC as specified in clause 6.3 of TS 33.503 [5].

The 5G ProSe UE-to-Network Relay replies to the Intermediate UE-to-Network Relay with the Relay Discovery Response.

4.	Upon receiving the Relay Discovery Response from the 5G ProSe UE-to-Network Relay, the Intermediate UE-to-Network Relay processes the received message using the discovery security materials associated with the RSC as specified in clause 6.3 of TS 33.503 [5]. If the processing is successful, the Intermediate UE-to-Network Relay updates the path information (e.g., hop count) and protects the updated message using the discovery security materials associated with the RSC as specified in clause 6.3 of TS 33.503 [5]. Then, the Intermediate UE-to-Network Relay replies to the 5G ProSe Remote UE with the message.

5.	Upon receiving the Relay Discovery Response from the Intermediate UE-to-Network Relay, the 5G ProSe Remote UE processes the received message using the discovery security materials associated with the RSC as specified in clause 6.3 of TS 33.503 [5].

**** END OF CHANGES ****